# IHO S-121
Encoding and 
Implementation Specification for 
Maritime Limits and Boundaries

Version 3.1

12 June 2017

# IHO S-121
Encoding and 
Implementation Specification for 
Maritime Limits and Boundaries

TABLE OF CONTENTS

1	Introduction	1

2	Explicit Text Encoding Format	3

3	Delimiting Structure	3

4	Implementation Model	10

5	Encoded Record Types	10

6	References Between Objects	31

7	Examples	33

Appendix  A.	The Carriage Return / Line Feed Delimiter	36

Appendix  B.	Abstract Syntax Notation	37

Appendix  C.	Implementation Schema	42

Appendix  D.	Encoding Tables	60

Appendix  E.	Record Oriented Encoding Structure  in Abstract Syntax Notation ASN.1	66

# IHO S-121
Encoding and 
Implementation Specification for 
Maritime Limits and Boundaries

# Introduction

Encoding is the translation of information from a structure, such as an internal database, into a format that can be communicated. The converse is decoding in which the transmitted information is converted into a form that can be used in the end application. Encoding establishes the interface at which data can be used.

Encoding is extremely important for S-121 Marine Limits and Boundaries because one of the primary uses of the data is in support of the legal description of States sovereign boundaries. As such MLB data must be easily readable in a judicial environment. This means that the data must be available in a human readable form. This form could be printed “charts” that can be easily viewed or it could be “lists of points” that can be easily read in printed form. Requiring a computer to assist in the reading introduces a variable that would undoubtedly be challenged in a judicial environment.  The corollary to this is that two people (i.e lawyers) reading the same document should take the same information away from it. Once you place a computing machine of any sort in between that process there is a risk that the machine alters the information however subtly. Any lawyer defending a position would immediately challenge the correctness of the computer system. While requiring a computer system to be able to read and interpret information isn’t normally a problem for text documents it’s a significant problem for geospatial data for numerous reasons. Spatial data cannot be easily visualized without tools. What this means is that there’s a need for the “submission encoding” which provides the DOALOS “List of Coordinates” be available in plain text that can be easily and directly read.

The S-121 Product Specification has defined three use cases. These document different uses of the data so they imply different encoding / decoding strategies. In summary the output formats required are:

1) Production Data – Production data represents a selection of a portion of an agency's Maritime Limit and Boundaries data holdings so that the data can be used in a different data product in a different context. An example is the extraction of official data representing the Contiguous Zone, Continental Shelf Area, Exclusive Economic Zone, and Territorial Sea for use in an Electronic Nautical Chart product. All of these zones and limits are separate objects in the S-121 Product Specification with attributes used to describe the legal source, Rights, Restrictions, Responsibilities and Parties involved. Only some of the attributes may need to be extracted for the depiction of the object in an S-101 defined ENC. This exchange may be internal to an agency or it may be between cooperating agencies. The exchange format may be a GIS system format, a custom XML format or GML. The detailed XML or GML schema will depend upon the exact content of the information required in the target product.

2) Legal Support Data – The output format required to support a legal declaration must be explicit and easy to read by non technical experts in a judicial or other similar environment and able to be used to support submission to DOALOS. The output may be verbose since only portions of it may ever be read at one time, but it needs to look closely like the information that is printed in treaties or national laws. This output must be textual so that there are no barriers to its understanding. It also needs to be simple.

XML is a form of structured text and it can be read, with some difficulty even by non-experts, so there may be situations where XML may be used in this context; however, it is not suited to be the format to be read in judicial or other formal proceedings. It is not simple and even experts get lost in all of the nested brackets, XLink references and other complexities of the XML syntax.

A structured text encoding is described in the following sub clauses that meet the requirements of being explicit and easy to read by not technical experts.

3) General Public Release Data – The release of data to the general public will be a subset of the data holdings respecting confidentiality agreements concerning data sources and other restrictions. This dataset may be reduced in resolution, and have limited metadata. However, the use of this data may be widespread. The appropriate output format would be GML or map images with XML metadata. These formats support a Marine Spatial Data Infrastructure (MSDI). Map images with XML metadata support the widely implemented Web Mapping Service (WMS). GML supports the more flexible Web Feature Service (WFS). For both these outputs parallel metadata information is required to support a Discovery Service. This involved implementation of the metadata required to drive the Catalogue Service for the Web (CSW). The detailed GML schema or map image plus XML schema will depend upon the detailed information that will be released and presented on a server such as an MSDI service or other public distribution service. Another avenue for public release would be the KML format. KML allows simple data to be overlaid on map backgrounds. This allows for very easy use of a generalized (lower resolution) of the data by the general public.

This document addresses the second Use Case of defining an explicit text format that is simple and easy to interpret in a judicial environment or to be used for submission of a “List of points” to DOALOS. The format is not limited to only points. Locations, Limits (and Boundaries) and Zones with their associated S-100 compliant geometric Points, Curves and Surfaces can be described. Support for Use Case 1 and 3 make use of existing published international standards.

# Explicit Text Encoding Format

A structured text encoding goes back in the history of computing to the early delimited formats that were suitable for record oriented media. Such formats are simple and easily readable. In early computer systems this simple approach was necessary because of the limitations of the early systems; however, a delimited record oriented approach has an advantage because it is very easy to understand. Each piece of information forms a record and delimiters identify fields within the records. The information structure needs to be flexible and non-limiting, and the data fields need to be of variable length. Each record needs to be numbered so that fields in one record can point to other records. This is required to implement the attribution by reference which supports the Rights, Restriction and Responsibility structure inherited from ISO 19152.

General numeric pointers may be conceptually simple but they can be difficult to follow. A naming structure is needed for labels to make the pointers easy to follow. Good practise for readability would organize the data so that relevant modifiers and attributes follow the primary record that they describe. Spatial information also needs to be explicit. Coordinates need to be described as readable numbers, and lists of coordinates need to be lists of readable numbers.

The resultant explicit text format needs to be printable with all of the information clearly available in the printed form. This means that there should be no hidden or “secret” unprintable characters. All the characters used should have a printable effect.

There are two aspects to the Explicit Text Encoding Format. The first is the description of the delimiting structure that allows records and sub records to be identified. The second is the description of the semantics of the format. This semantics is described using the Abstract Syntax Notation (ASN.1) which is a widely used method for describing a syntax that has been standardized by ISO as standard ISO 8824.

# Delimiting Structure

The earliest record oriented encoding format used punch cards where the length of a card was a fixed 80 characters. Data communications systems soon emerged after the era of the punch card and allowed variable length records. This required the use of a terminator to delimit the end of a record. Appendix A gives some background on why the sequence of CR (IA5 Code 13) followed by Line Feed LF (IA5 Code 10) character combination (CR LF) became the almost universal delimiter for an unstructured character string. Other systems that support CR or LF alone generally also support the CR LF combination. The use of the CR LF unstructured character string delimiter allows the S-121 Explicit Text Format to be read by almost all text editors and directly printed.

The intent of the S-121 Explicit Text Format is to allow any user to read, print and interpret a set of S-121 data without the need for special tools. At the minimum data can be “dumped” using a tool such as HexDump (or one of its many equivalents) and the actual bytes of data can be read in hexadecimal and in text. Even at this level the text can be read by non-experts. Text processing software such as the simple “Microsoft Notebook” program or a word processor such as “Microsoft Word” or equivalent can easily read text delimited by a CR LF.

The S-121 Explicit Text Format is also parsable. That means that the data can be unambiguously read by a computer program and parsed allowing for it to be decoded into the original data structure from which it was generated.

The S-121 Explicit Text Format is record oriented. A record often corresponds to a character string, but the two are not identical. The record structure is described below. Sometimes there may be more than one character string within a record since some attributes take on a character string value type and a field of text may contain “paragraphs” that each are a separate string.

## Record Structure

A Record is a structured block of information. There are two types of records: Identified Records and Special Records. The structure of both types of records is the same except that an identified record includes an identifier field so that the record may be referenced. The two types of records are shown in Figure 1. An example of an identified record might be a feature such as an “Outer Limit of the Territorial Sea” object that may have a record number of, for example “Limit-439”.  An example of a special record might be a metadata record, such as the title of the particular data set. Since there is only one title the record can be uniquely identified by the Record Type Identifier of, for example, “Title”.

Figure 1 – Record Structure

There also needs to be a record terminator (an End of Record Marker) which is explicit and which cannot appear anywhere in the data. In addition, there is a need for a terminator for sub records.

An Identified Record needs to begin with a record number.

The Record Number is unique for each identified record in a data file. For those record types that require a unique identifier uID (as defined in ISO 19152) the Record Number serves as the uID with the namespace corresponding to the type of data record. That is the Record Number is a structured entity. The substructure of a Record Number is a namespace identifier followed by a dash character “-“ followed by a number. For example, “Zone-3” has the namespace of Zone followed by a delimiter and then a number identifying a particular zone (number 3). Figure 2 illustrates the sub structure of a Record Number.

Figure 2 – Record Number

The Record Type Identifier is the descriptor of the record type. There is a fixed list of record types identified in the syntax. This includes special types such as a metadata record or the End of File. If the record describes a feature instance the Record Type Identifier would describe the type of the feature. Feature types are explicitly defined for S-121 and recorded in the Feature Catalogue. For example, an Exclusive Economic Zone feature type has the name recorded in the Feature Catalogue of “Exclusive Economic Zone”. This name text string is a unique identifier of the type. It is more verbose that the 6 character alpha code “EXEZNE” or the camelCaseCode “ExclusiveEconomicZone” but it is preferable because it is easy to read in a judicial environment by non-technical experts.

The Record Contents contain the attributes describing the record type. The attributes that are applicable are described in the syntax and are record type dependant. These attributes form sub records and the sub records need to be separated by sub record delimiters. The sub records may include references to other records using the unique identifier uID of the referenced record. For example a reference to legal sources may be to “Source-23” and to “Source-14” and a reference to the spatial geometry may be to “Curve-92, Curve-47, and Curve-17”.

A delimiter is needed to separate the sub elements of the record structure.

The Horizontal Tab TAB (IA5 Code 09) is the record field delimiter. This delimits between the Record Number, the Record Type, and the Record Contents fields. It may also be used to delimit sub fields within the record contents. The use of the TAB delimiter as a sub record delimiter makes the S-121 format into a TAB Separated Values (TSV) type of file.  To enhance readability (to line-up fields when printing) the LF (IA5 Code 10) may be used together with a TAB character as a TAB LF pair. This will be treated as equivalent to a single TAB.

The end of record must be unique.

The End of Record Marker is the primary delimiter of the explicit textual data format. It is defined as the TAB (IA5 Code 09) followed by the Carriage Return CR (IA5 Code 13) followed by Line Feed LF (IA5 Code 10) character combination. The TAB CR LF character combination is unique and has a clear effect when the data is printed. Some conventional word processing tools may generate one line spacing and others may print as two lines. Both are clear and will mean the same to a human reader.

The use of the TAB CR LF character combination as the End of Record Marker also permits the use of LF or CR or CRLF within an arbitrary text string (paragraph marker). The use of LF for this purpose is preferred since it is compatible with more text tools, although any of LF or CR or CRLF would work.

## End of File

The S121 record oriented format needs an explicit unique end of file. This is shown in Figure 3. There is only one End of File per file and it needs to be easily readable text. The text string === End of File === is used. This special record consists of only a Record Type Identifier and an End of Record Marker.

Figure 3 – End of File Record

Some computer systems require another terminator such as the Control Character Cntrl Z (IA5 Code 26) as used in UNIX files. Any additional terminators outside of the S-121 data file are the responsibility of the operating system managing the data. They do not affect the data syntax.

## Other Sub Record Delimiters

In order for the fields within a record to be clear there need to be explicit sub-record delimiters. Allowing for variable length data fields requires that the delimiters be imbedded in the data. This requires some special characters and data strings to be reserved. These delimiters must be printable characters that can be seen in the text. If in the very rare case that a character string appears in the data that mimics a delimiter an escape mechanism is required to allow the rare occurrence to be expressed.

The delimiters identified are:

| TAB (IA5 Code 09) | - Record Field Separator |
| --- | --- |
| - (IA5 Code 45) | - Record Identifier Sub-field Separator (used to separate the namespace from the ID number in a record identifier) |
| = (IA5 Code 61) followed by an optional Space or multiple Space characters (IA5 Code 32) | - Sub-field Separator (used separate the attribute type from the attribute value sub field) The use of optional Space characters makes the data more readable. The use of at least one is recommended. |
| , (IA5 Code 44) followed by an optional Space or multiple Space characters (IA5 Code 32) | - Sub-sub-field Separator (used for some attributes that contain strings of values) The use of optional Space characters makes the data more readable. The use of at least one is recommended. |
| / (IA5 Code 47) followed by an optional Space or multiple Space characters (IA5 Code 32) | - Sub-sub-field Lat/Long Separator (used to for some attributes that contain Lat / Long values) The use of optional Space characters makes the data more readable. |

The Tab character is the primary delimiter. If a Tab character must be inserted in a text string (and there is very little reason to do so) then it can be replaced by the string <TAB>.

The tab character delimits fields. In some cases there is a need to identify sub-fields. In this case the delimiter = is used. The character = would separate a sub-field header from a value.

The character / would separate two numbers in a numeric attribute such as a Latitude and Longitude. The use of subfields is limited to certain record types as identified in the syntax.

Some subfields, such as attributes may take on a string of attribute values. These attribute values may be separated by a sub-sub-field delimiter   ,   .

Figure 3 shows the general record structure from Figure 1 as a text string with imbedded TAB character delimiters.

Figure 4 – Delimited Record

This record structure has the advantage that it can be read by common office word processing software and easily read into a spread sheet as a Tab Separated Value (TVS) file. The only variation is the allowance for Line Feed characters within the body of the record for certain types of records that contain an unstructured text field.

To make the information more readable extraneous SPACE characters (IA5 Code 32) should be ignored except in attributes that are character strings (text). That is, two or more SPACE characters should be treated as a single SPACE character. This allows extra SPACE characters to be inserted in a file to improve readability without changing the meaning of the data.

The Record Contents may be sub-delimited by use of the TAB character. The Structure of the Record Contents consists of a string of attributes. Each attribute is defined with a unique type name followed by the sub-field delimiter = followed by an attribute value field. If an attribute type is optional in the model it can be omitted for instances of the data. If an attribute type is permitted to be repeated in the model then the attribute type name followed by the sub-field delimiter = followed by an attribute value field may be repeated.

Some attribute value fields allow for multiple values. The sub-sub-field delimiter   ,    may be used to separate these values.

If a string of values is coordinate data, such as a string of vertices describing a curve, the Sub-sub-field Lat/Long Separator / would separate two numbers. For example a string of coordinates may be Lat1/Long1, Lat2/Long3, Lat3/Long3, …..  .

Figure 5 shows a record using delimiters and sub field delimiters.

Figure 5 – Delimited and Sub Delimited Record

# Implementation Model

The S-121 Application Schema is defined as part of the Product Specification and is a UML model of all of the objects and associated attributes that are required to support Maritime Limits and Boundaries. The model is derived from both S-100 and ISO 19152 Land Domain Administrative Model. The Application Schema is hierarchical and has many classes that invoke other classes to form a logical tree of information elements. This is the way the models in S-100 and in ISO work. However, to implement the model, especially in a record oriented structure, it becomes very difficult to use such a hierarchical structure. In Appendix C an implementation model has been defined that flattens the structure so that there is only one information class to implement for each information type. Appendix C shows both the base classes of the Application Schema and the derived Implementation Classes with the implementation relationship.

In the Implementation Model all of the attribute values have been converted to character strings. These may be general character strings or structured character strings or may take on specific code list values as character strings. Numerical values have been converted into text. Relationships between classes have been made navigable and may be implemented as pointers that reference the Record Number or the associated object. These relationships are also shown in the model in Appendix C.

# Encoded Record Types

The following Identified Record Types have been defined. The manner in which they are used is described in the syntax of the format. These record types correspond to the instantiable types from the Implementation Model. Each type may have a different structure and has a different namespace for the Record Number.

Location Feature Type Record

Limit Feature Type Record

Zone Feature Type Record

Space Feature Type Record

Point Spatial Attribute Record

Curve Spatial Attribute Record

Surface Spatial Attribute Record

Basic Administrative Unit Information Object Record

Right Information Object Record

Restriction Information Object Record

Responsibility Information Object Record

Party Group Information Object Record

Spatial Source Information Object Record

Administrative Source Information Object Record

The encoding for these identified record identifiers is provided in table D1 in Appendix D.

The following Special Record Types have been defined. These types do not have a Record Number. Each may occur only once in a data file and their Record Type Identifier is sufficient to distinguish them.

Metadata - Title

Metadata - S-100 version

Metadata - S121 version

Metadata - S121 version date

Metadata - S121 language

Metadata - Security classification

Metadata - S121 Contact

Metadata - S121 specification URL

Metadata - S121 identifier  (“S-121”)

Metadata - S121 maintenance

End of File Record

The encoding for these special record identifiers is provided in table D2 in Appendix D.

## Record Type Names and Attribute Names

The Record Type Identifier that is inserted in the record following the record number needs to be understandable by a person reading a printout of the encoded data. The object names used in the model are not appropriate since they were designed to support the modelling. This is also true for attribute names which are chosen in the model to be unique within the context of all of the other S-100 standards and ISO TC211 Geographic Information standards. That is, they are cryptic so that they work smoothly within the model and contain extra lead-in characters such as “MA_” so that they are non-interfering with all of the IHO and ISO namespaces.

A set of encoding tables are provided in Appendix D to translate from the Record Type Identifier used in the model to a easily readable Record Type Identifier. Similarly a translation table is provided for the Special Record Type names to readable names.

The attributes built-into the feature types have cryptic unique UML names. A translation table is also provided to convert these Attribute identifiers to readable names.

The attribute values are character strings, structured character strings or code list values expressed as character strings. The code list values for the are as described in the model.

Feature records have a feature unit name, such as Exclusive Economic Zone, that is registered in the S-100 object register and also in the Feature Catalogue for S-121. These features have a cryptic code (alpha code or camelCaseCode) which is not suitable for use in an explicit text format. The identifiers that need to be used in the explicit text format are the names of the features types. These names are also registered in the S-100 object register and also in the Feature Catalogue for S-121. These have been chosen to be unique and suitable for presentation in a legal environment.

Registered attributes may also be used with feature records. Again these are registered in the S-100 object register and also in the Feature Catalogue for S-121, and each feature in the feature catalogue indicates which attributes may be used with it. These attributes are used in the same way as the built-in attributes with the value sub-field separated using the delimiter =.

## Feature Type Record

The set of Feature Type Records allows one to express the features and attributes identified in S121 Feature Catalogue.

There are two types of attributes for Feature Type Records. Thematic attributes are defined in the Feature Catalogue and may be used with each specific Feature Type. Built-in attributes are part of the objects that describe the components of the Application Schema and the Implementation Model. The built-in attributes are described in the Implementation Classes in Appendix C. The built-in attributes have cryptic names used in the UML model. A translation from the cryptic names to readable names is provided in Table D3 in Appendix D.

The listed values in the Feature Catalogue provide attribute values. The Feature Catalogue is described in a separate document.

There are four types of Feature Records:

Location Feature Type Record

Limit Feature Type Record

Zone Feature Type Record

Space Feature Type Record

Each has a specific structure and defines a unique namespace for the Record Number. These are the namespace identifier followed by a dash character “-“ followed by an id number:

Location-idNumber

Limit-idNumber

Zone-idNumber

Space-idNumber

The name of the Feature Record is taken from the Feature Concept Dictionary and Feature Catalogue )for the particular feature from the name of the feature. Name is used rather than code because the name is more human readable. The names of the MLB defined features have deliberately been made clear text so that they can appear in the explicit text format and be understood without the need for the reader to look up the meaning. For example the territorial sea object has the name “Territorial Sea” while its code is “TESARE”. The name is used in the Record Type Identifier field.

### Location Feature Type Record

The Location Feature Type record has the following structure:

Figure 6 – Location Record

The Location record type allows the following attributes:

MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

MA_orgtype OPTIONAL,	-- Computational origin of the element: defined, densified, computed or construction

MA_interpolationRole,	-- Computational origin of the element: defined, densified, computed or construction

MA_pointType OPTIONAL,	-- The type of point

TransformedLocation OPTIONAL,	-- Location of transformed point

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

In addition the attribute types defined in the Feature Catalogue for Location Type Features may be used.

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Limit Feature Type Record

The Limit Feature Type record has a similar structure. The difference is in the namespace for the Record Number field, the Record Type Identifier which is taken from the set of Limit type features in the Feature Catalogue and the list of allowable attributes.

Figure 7 – Limit Record

The Limit record type allows the following attributes:

MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

MA_arctyp OPTIONAL,	-- Type of computation used to define an arc (line) geodesic or loxodrome

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

In addition the attribute types defined in the Feature Catalogue for Limit Type Features may be used.

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Zone Feature Type Record

The Zone Feature Type record also has a similar structure. The difference is in the namespace for the Record Number field which is Zone, the Record Type Identifier which is taken from the set of Zone type features in the Feature Catalogue and the list of allowable attributes.

Figure 8 – Zone Record

The Zone record type allows the following attributes:

MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

MA_verdom OPTIONAL,	-- Category of vertical domain of the object delimited.

SEQUENCE OF {MA_areaSize} OPTIONAL,	-- The area value is represented as a character string value in a Unit of Measure.

MA_areaType OPTIONAL,	-- Area type: official area, non-official area, calculated area, surveyed area

MA_referencePoint OPTIONAL,	-- The coordinates of a point inside the spatial unit

MA_surfaceRelation OPTIONAL,	-- Relationship to surface

MA_UnitOfMeasure OPTIONAL,	-- Units used to describe area size

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

In addition the attribute types defined in the Feature Catalogue for Zone Type Features may be used.

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Space Feature Type Record

The Space Feature Type record differs in that there are no Space defined records in the Feature Catalogue. However, the record structure similar. The difference is in the namespace for the Record Number field which is Space, the Record Type Identifier is from Space records constructed using the generic Space feature type from the Feature Catalogue.

Figure 9 – Space Record

The Space record type allows the following attributes:

MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

MA_verdom OPTIONAL,	-- Category of vertical domain of the object delimited.

MA_referencePoint OPTIONAL,	-- The coordinates of a point inside the spatial unit

SEQUENCE OF {MA_volumeSize} OPTIONAL,	-- The volume size value is represented as a character string value in a Unit of Measure.

MA_UnitOfMeasure OPTIONAL,	-- Units used to describe volume size

MA_volumeType OPTIONAL,	-- Volume type: official volume, non-official volume, calculated volume, surveyed volume

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

In addition the attribute created for use with the generic Space record may be used.

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

## Geometry Type Records

The set of Geometry Type Records describes the geometry associated with the Feature Records in terms of the Geometry types defined in S-100. These types are:

- Point,

- Curve

- Surface

The point describes a Latitude / Longitude direct position. Other attributes describe the characteristics of the point. Latitude and Longitude may be described in either decimal degrees or in Degrees, Minutes and Seconds.

The Curve consists of a set of vertices that are also Latitude / Longitude direct positions.

The Point direct positions and the vertices are the only actual geometric values expressed. The rest of the geometry is defined through feature to feature relations that allow a limit defined by a curve to be bound by a location defined by a point. A zone is bounded by a set of limits described by curves. A Space is defined by a zone described by a surface with an attribute of height. This is expressed as a point and a distance (elevation).

The Point, Curve and Surface objects each have attributes inherited from S100 and S121 Spatial attribute type. One important attribute is locationByText. This allows a textual description of a spatial object. For example one boundary may not be well known so the limits bounding a zone may be several detailed geometric sets of vertices represented by limits and one limit represented by a textual description that allows closure of the surface without obliging one to specify geometric information where it is not known or not officially established.

The built-in attributes are described in the Implementation Classes in Appendix C. The built-in attributes have cryptic names used in the UML model. A translation from the cryptic names to readable names is provided in Table D3 in Appendix D.

### Point Type Record

The Point Type is illustrated in Figure 10. It differs from the other Identified records in that the Record Type Identifier is not used.

Figure 10 – Point Record

The Point record type allows the following attributes:

MA_PointGeometry,	-- Lat / Long of a Direct Position.

MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model. For Geometry type records this is a relationship to SpatialSource.

### Curve Type Record

The Curve Record Type is illustrated in Figure 11. Similarly to the Point record the Record Type Identifier is not used.

Figure 11 – Curve Record

The Curve record type allows the following attributes:

MA_CurveGeometry,	-- A set of vertices( Lat / Long Direct Position) or an Arc by Centre Point or a Circle By Centre Point.

MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model. For Geometry type records this is a relationship to SpatialSource.

### Surface Type Record

The Surface Record Type is illustrated in Figure 12. The Record Type Identifier is not used. A Surface is defined by the Curves corresponding to Limits that bound it. This is expressed through the feature to feature relationships between the Zone and Limit features. The Surface Record only carries attributes about the surface.

Figure 12 – Surface Record

The Surface record type allows the following attributes:

MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model. For Geometry type records this is a relationship to SpatialSource.

### Volume Type Record

The Volume Record Type is illustrated in Figure 13. The Record Type Identifier is not used. A Volume is defined by a Surface corresponding to a Zone with a vertical extent. The height is expressed by a point and a distance. The Volume Record describes a position and a vertical extent distance at that point.

Figure 13 – Volume Record

The Volume record type allows the following attributes:

MA_verticalPosition,	-- Start level for vertical extent

MA_verticalExtent,	-- Describes Vertical extent of space

MA_UnitOfMeasure OPTIONAL,	-- Describes Unit of measure of the vertical extent

MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model. For Geometry type records this is a relationship to SpatialSource.

## Information Object Record

The object structure for legal objects derived from ISO 19152 makes use of the Information Objects from S-100. The record structure is that of an Identified Record since  Information Object records may be identified. Information Object types are included in the Feature Catalogue. The field Record Type is populated by the Name field from the Feature Catalogue entry. This is a human readable field.

### BAUnit Type Record

The Basic Administrative Unit is an information object to “which (one or more) unique and homogeneous rights, responsibilities or restrictions are associated”. It is an information object since it does not directly take on spatial attributes. The Basic Administrative Unit Record Type is illustrated in Figure 14. The Record Type Identifier is “Administrative Unit” from the Name field in the Feature Catalogue entry for BAUnit.

Figure 14 – BAUnit Record

The BAUnit record type allows the following attributes:

MA_BAtype,	-- the use type of a BAUnit: SovereigntyUnit, SoverignRightsUnit, JointDevelopmentUnit, OtherRegulatoryUnit

MA_Context OPTIONAL,	-- allows the context for an instance of a BAUnit to be described.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

In addition the attribute types defined in the Feature Catalogue for the BAUnit Type Features may be used.

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Right Type Record

The S121_Right is an action, activity or class of actions that a system participant may perform on or using an associated resource. It is an information object since it does not directly take on spatial attributes. The Right Record Type is illustrated in Figure 15. The Record Type Identifier is “Administrative Right” from the Name field in the Feature Catalogue entry for S121_Right.

Figure 15 – Administrative Right Record

The Right record type allows the following attributes:

MA_RightType,	-- the type of the right: sovereignty, sovereignRight, accessRight, harvestRight, easementRight

MA_RRRdescription OPTIONAL,	-- description regarding the right.

MA_share OPTIONAL,	-- share of an instance of a right

MA_shareCheck OPTIONAL,	-- boolean indicating whether the constraint is applicable

MA_timeSpec OPTIONAL,	-- operational use of a right in time sharing.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Restriction Type Record

The S121_Right is a formal or informal entitlement to refrain from doing something.. It is an information object since it does not directly take on spatial attributes. The Restriction Record Type is illustrated in Figure 16. The Record Type Identifier is “Administrative Restriction” from the Name field in the Feature Catalogue entry.

Figure 16 – Administrative Restriction Record

The Restriction record type allows the following attributes:

MA_partyRequired,	-- indicates whether a party is required for the registration of the restriction in the association to Party

MA_RestrictionType,	-- the type of the Restriction: timeBasedRestriction, passageRestriction, accessRestriction, useRestriction, jurisdictionRestriction, resourceRestriction

MA_RRRdescription OPTIONAL,	-- description regarding the right, restriction or responsibility.

MA_share OPTIONAL,	-- share of an instance of a right

MA_shareCheck OPTIONAL,	-- boolean indicating whether the constraint is applicable

MA_timeSpec OPTIONAL,	-- operational use of a right in time sharing.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Responsibility Type Record

The S121_Responsibility is a formal or informal obligation to do something. It is an information object since it does not directly take on spatial attributes. The Responsibility Record Type is illustrated in Figure 17. The Record Type Identifier is “Administrative Responsibility” from the Name field in the Feature Catalogue entry

Figure 17 – Administrative Responsibility Record

The Responsibility record type allows the following attributes:

MA_ResponsibilityType,	-- the type of the responsibility: maintenanceResponsibility

MA_RRRdescription OPTIONAL,	-- description regarding the right, restriction or responsibility.

MA_share OPTIONAL,	-- share of an instance of a right

MA_shareCheck OPTIONAL,	-- boolean indicating whether the constraint is applicable

MA_timeSpec OPTIONAL,	-- operational use of a right in time sharing.

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Party Type Record

The S121_Party is a person or organisation that plays a role in a rights transaction. The Party Record Type is illustrated in Figure 18. The Record Type Identifier is “Party” from the Name field in the Feature Catalogue entry

Figure 18 – Party Record

The Party record type allows the following attributes:

exPID,	-- the identifier of the party in an external registration

MA_PartyRole OPTIONAL,	-- the role of the party

MA_partyType,	-- the type of the party

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Group Party Type Record

The S121_GroupParty is any number of parties, forming together a distinct entity, with each party registered. The Group Party Record Type is illustrated in Figure 19. The Record Type Identifier is “Group Party” from the Name field in the Feature Catalogue entry

Figure 19 – Group Party Record

The Group Party record type allows the following attributes:

MA_groupPartyType,	-- the type of the group party: agreement, association

exPID,	-- the identifier of the party in an external registration

MA_PartyRole OPTIONAL,	-- the role of the party

MA_partyType,	-- the type of the party

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Spatial Source Type Record

The S121_SpatialSource provides documentation of the source spatial Attributes of the referenced information. The S121_Spatial Source Implementation Object aggregates all of the attributes from the S121_SpatialSource structure in one class. The Spatial Source Record Type is illustrated in Figure 20. The Record Type Identifier is “Spatial Source” from the Name field in the Feature Catalogue entry

Figure 20 – Spatial Source Record

The Spatial Source record type allows the following attributes:

MA_SpatialSourceType OPTIONAL,	-- The type of spatial source document

SEQUENCE OF {MA_SourceORlinkageURL} OPTIONAL,	-- Location for online access

SEQUENCE OF {MA_SourceORprotocol OPTIONAL,	-- Online Resource Protocol

SEQUENCE OF {MA_SourceORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

MA_SourceORname OPTIONAL,	-- Name of the resource

MA_SourceORdescription OPTIONAL,	-- Description of what the resource is/does

MA_SourceORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

MA_RegistryNumber OPTIONAL,	-- Unique official identifier of the record in a registry

MA_lifeSpanStamp OPTIONAL,	-- The moment that the event represented by the instance of LA_Source is further processed in the LA system

MA_acceptance OPTIONAL,	-- The date of force of law of the source by an authority

MA_maintype OPTIONAL,	-- The physical type of document

MA_availabilityStatus OPTIONAL,	-- The availability status of document

MA_submission OPTIONAL,	-- The date of submission of the source by a party

MA_recordation OPTIONAL,	-- The date of registration (recordation) of the source by registering authority

MA_RPindividualName OPTIONAL,	-- Name of the responsible individual

MA_RPcontactORlinkageURL OPTIONAL,	-- Responsible party location for online access

MA_RPcontactORprotocol OPTIONAL,	-- Responsible party connection protocol to be used

MA_RPcontactORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

MA_RPcontactORname OPTIONAL,	-- Name of resource

MA_RPcontactORdescription OPTIONAL,	-- Description of what the resource is/does

MA_RPcontactORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

MA_RPorganizationName OPTIONAL,	-- Name of the responsible organization

MA_RPpositionName OPTIONAL,	-- Position of the responsible person

MA_RPcontactPhone OPTIONAL,	-- Telephone numbers at which the organisation or individual may be contacted

MA_RProle OPTIONAL,	-- Function performed by the responsible party

MA_RPcontactAddressCountry OPTIONAL,	-- Country of the physical address

MA_RPcontactAddressDeliveryPoint OPTIONAL,	-- Address line for the physical address (Street name, box number, suite)

MA_RPcontactAddressCity OPTIONAL,	-- City of the physical address

MA_RPcontactElectronicMailAddress OPTIONAL,	-- Electronic Mail address

MA_RPcontactAddressAdministrativeArea OPTIONAL,	-- State, province of the physical address

MA_RPcontactAddressPostalCode OPTIONAL,	-- Postal Code for the physical address

SEQUENCE OF{DQnameOfMeasure} OPTIONAL,	-- Data Quality name of measure

DQmeasureIdentification OPTIONAL,	-- Data Quality measure identification

DQmeasureDescription OPTIONAL,	-- Data Quality measure description

DQevaluationMethodType OPTIONAL,	-- Data Quality evaluation method type

DQevaluationMethodDescription OPTIONAL,	-- Data Quality evaluation method description

DQevaluationProcedure OPTIONAL,	-- Data Quality evaluation method type

SEQUENCE OF {DQdateTime} OPTIONAL,	-- Data Quality reference date

SEQUENCE OF {DQresult},	-- Data Quality result

MA_ExternalArchiveAcceptance OPTIONAL,	-- The date of force of law of the source by the authority

MA_ExternalArchiveData OPTIONAL,	-- The content of the source

MA_ExternalArchiveExtraction OPTIONAL,	-- Date of retrieval of information from an external archive

MA_ExternalArchiveRecordation OPTIONAL,	-- Date information was recorded in an external archive

MA_ExternalArchive_sID OPTIONAL,	-- Identifier of the external archive

MA_ExternalArchiveSubmission OPTIONAL,	-- Date of submission of information in an external archive by a party

MA_groupPartyType,	-- the type of the group party: agreement, association

exPID,	-- the identifier of the party in an external registration

MA_PartyRole OPTIONAL,	-- the role of the party

MA_partyType,	-- the type of the party

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

### Administrative Source Type Record

The S121_AdministrativeSource is a source with the administrative description (where applicable) of the parties involved, the rights, restrictions and responsibilities created and the basic administrative units affected. The S121_Admiministrative Source Implementation Object aggregates all of the attributes from the S121_AdministrativeSource structure in one class. The Administrative Source Record Type is illustrated in Figure 21. The Record Type Identifier is “Administrative Source” from the Name field in the Feature Catalogue entry

Figure 21 – Administrative Source Record

The Administrative Source record type allows the following attributes:

MA_AdministrativeSourceType OPTIONAL,	-- The type of administrative source document

SEQUENCE OF {MA_SourceORlinkageURL} OPTIONAL,	-- Location for online access

SEQUENCE OF {MA_SourceORprotocol OPTIONAL,	-- Online Resource Protocol

SEQUENCE OF {MA_SourceORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

MA_SourceORname OPTIONAL,	-- Name of the resource

MA_SourceORdescription OPTIONAL,	-- Description of what the resource is/does

MA_SourceORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

MA_RegistryNumber OPTIONAL,	-- Unique official identifier of the record in a registry

MA_lifeSpanStamp OPTIONAL,	-- The moment that the event represented by the instance of LA_Source is further processed in the LA system

MA_acceptance OPTIONAL,	-- The date of force of law of the source by an authority

MA_maintype OPTIONAL,	-- The physical type of document

MA_availabilityStatus OPTIONAL,	-- The availability status of document

MA_submission OPTIONAL,	-- The date of submission of the source by a party

MA_recordation OPTIONAL,	-- The date of registration (recordation) of the source by registering authority

MA_RPindividualName OPTIONAL,	-- Name of the responsible individual

MA_RPcontactORlinkageURL OPTIONAL,	-- Responsible party location for online access

MA_RPcontactORprotocol OPTIONAL,	-- Responsible party connection protocol to be used

MA_RPcontactORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

MA_RPcontactORname OPTIONAL,	-- Name of resource

MA_RPcontactORdescription OPTIONAL,	-- Description of what the resource is/does

MA_RPcontactORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

MA_RPorganizationName OPTIONAL,	-- Name of the responsible organization

MA_RPpositionName OPTIONAL,	-- Position of the responsible person

MA_RPcontactPhone OPTIONAL,	-- Telephone numbers at which the organisation or individual may be contacted

MA_RProle OPTIONAL,	-- Function performed by the responsible party

MA_RPcontactAddressCountry OPTIONAL,	-- Country of the physical address

MA_RPcontactAddressDeliveryPoint OPTIONAL,	-- Address line for the physical address (Street name, box number, suite)

MA_RPcontactAddressCity OPTIONAL,	-- City of the physical address

MA_RPcontactElectronicMailAddress OPTIONAL,	-- Electronic Mail address

MA_RPcontactAddressAdministrativeArea OPTIONAL,	-- State, province of the physical address

MA_RPcontactAddressPostalCode OPTIONAL,	-- Postal Code for the physical address

SEQUENCE OF{DQnameOfMeasure} OPTIONAL,	-- Data Quality name of measure

DQmeasureIdentification OPTIONAL,	-- Data Quality measure identification

DQmeasureDescription OPTIONAL,	-- Data Quality measure description

DQevaluationMethodType OPTIONAL,	-- Data Quality evaluation method type

DQevaluationMethodDescription OPTIONAL,	-- Data Quality evaluation method description

DQevaluationProcedure OPTIONAL,	-- Data Quality evaluation method type

SEQUENCE OF {DQdateTime} OPTIONAL,	-- Data Quality reference date

SEQUENCE OF {DQresult},	-- Data Quality result

MA_ExternalArchiveAcceptance OPTIONAL,	-- The date of force of law of the source by the authority

MA_ExternalArchiveData OPTIONAL,	-- The content of the source

MA_ExternalArchiveExtraction OPTIONAL,	-- Date of retrieval of information from an external archive

MA_ExternalArchiveRecordation OPTIONAL,	-- Date information was recorded in an external archive

MA_ExternalArchive_sID OPTIONAL,	-- Identifier of the external archive

MA_ExternalArchiveSubmission OPTIONAL,	-- Date of submission of information in an external archive by a party

MA_groupPartyType,	-- the type of the group party: agreement, association

exPID,	-- the identifier of the party in an external registration

MA_PartyRole OPTIONAL,	-- the role of the party

MA_partyType,	-- the type of the party

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

Also additional attributes are used to represent the relations between a record and other identified records. These attributes are described in the relationship model.

# References Between Objects

References may occur between Information Object Records implementing the Rights, Restriction, Responsibility and Party structure and with Feature Object Records. These references make use of the ID on the record types. The references are treated as another attribute with the attribute type defined by the role identified for the relation. These relations are given in the model such as is illustrated in Figure C2 and C3. An example would be a Right record referencing an associated Administrative Source record by the rightSource relation.

The list of relation attributes are:

Right - 	-- Reference from Information Object Records of type S121_BAUnit to S121_Right.

Restriction - 	-- Reference from Information Object Records of type S121_BAUnit to S121_Restriction .

Responsibility - 	-- Reference from Information Object Records of type S121_BAUnit to S121_Responsibility .

BAUnit As Party -	-- Reference from Information Object Records of type S121_Party Record to S121_Party.

Members	-- Reference from Information Object Records of type S121_GroupParty to S121_Party.

Members	-- Reference from Information Object Records of type S121_GroupParty to S121_Party.

Zone Admin Attribute	-- Reference from Information Object Records of type Zone to S121_BAUnit.

Space Admin Attribute	-- Reference from Information Object Records of type Space to S121_BAUnit..

Spatial Point	-- Reference from Information Object Records of type Location to S121_Point.

Spatial Curve	-- Reference from Information Object Records of type Curve to S121_Curve.

Spatial Zone	-- Reference from Information Object Records of type Zone to S121_Surface.

Spatial Space	-- Reference from Information Object Records of type Space to S121_Volume.

Points	-- Reference from Information Object Records of type Limit to Points

Minus Boundary	-- Reference from Information Object Records of type Zone to Limit

Plus Boundary	-- Reference from Information Object Records of type Zone to Limit

Vertical Extent	-- Reference from Information Object Records of type Space to Zone

Relation BAUnit	-- Reference from Information Object Records of type BAUnit to BAUnit

- These relations are added to the attributes of a particular object.

## Source and Spatial Source

Every object may include a reference source. For Right, Restriction, Responsibility and Party objects this is done through the S121_AdministrativeSource information object. For the S121_FeatureUnit and its subtypes and for S121_SpatialAttributeType and its subtypes this is done through a reference to S121_SpatialSource. These attributes are also added to the associated objects.

The list of relation attributes pertaining to source are:

Unit Source- 	-- Reference from Information Object Records of type S121_BAUnit to S121_AdministrativeSource .

Conveyancer Source 	-- Reference from Information Object Records of type S121_Party to S121_AdministrativeSource

Right Source 	-- Reference from Information Object Records of type S121_Right to S121_AdministrativeSource

Right Source 	-- Reference from Information Object Records of type S121_Right to S121_AdministrativeSource

Responsibility Source 	-- Reference from Information Object Records of type S121_Responsibility to S121_AdministrativeSource

Restriction Source 	-- Reference from Information Object Records of type S121_Restriction to S121_AdministrativeSource

Feature Unit Source	-- Reference from Information Object Records of type S121_FeatureUnit to S121_SpatialSource.

Location Unit Source	-- Reference from Information Object Records of type Location to S121_SpatialSource.

Limit Unit Source	-- Reference from Information Object Records of type Limit to S121_SpatialSource.

Zone Unit Source	-- Reference from Information Object Records of type Zone to S121_SpatialSource.

Space Unit Source	-- Reference from Information Object Records of type Space to S121_SpatialSource.

Spatial Attribute Source	-- Reference from Information Object Records of type S121_SpatialAttributeType to S121_SpatialSource.

Point Source	-- Reference from Information Object Records of type S121_Point to S121_SpatialSource.

Curve Source	-- Reference from Information Object Records of type S121_Curve to S121_SpatialSource.

Surface Source	-- Reference from Information Object Records of type S121_Surface to S121_SpatialSource.

Volume Source	-- Reference from Information Object Records of type S121_Volume to S121_SpatialSource.

Composite Object Spatial Source	-- Reference from Information Object Records of type S121_Composite to S121_SpatialSource.

## Versioned Objects

All of the S-121 objects may be versioned. Versioning is simply another attribute, defined in the feature catalogue as an attribute that can be used with any object. There are two versioning attributes:

beginLifespanVersion,	-- Start time of a specific instance version

endLifespanVersion OPTIONAL,	-- End time of a specific instance version

The attribute “beginLifespanVersion” is optional. When it is missing it means that the associated object is currently valid.

The current version of the S121 makes the attribute “beginLifespanVersion” mandatory. This means it appears in all records. This is the way the attribute is handled in ISO 19152. This is the appropriate approach in a database where every record may be different, but maybe not in a dataset. Making this attribute mandatory may be an unacceptable overhead in that it makes the data more verbose and may also reduce readability by presenting the same information over and over again. A potential alternative is to establish a default begin date in the metadata and make the attribute optional.

# Examples

The use of explicit text records with tab delimiters will make the data very readable. The following are a few simple examples of some records.

## Example Simple Boundary Points

The following is an example simple dataset consisting of only two control points. The lead-in metadata for the example dataset has not been included. This example contains only two types of records:

- Location Feature Type Record

- Point Spatial Attribute Record

Note that since there are no zone or space objects there are no administrative objects.

Location-1	Baseline Point	Computational origin = defined	
Feature context = East end of baseline	
Releasability= official	
Beginning of validity=2017:01:01	
Spatial Point= Point-1

Location-2	Baseline Point	Computational origin = defined	
Feature context = West end of baseline	
Releasability= official	
Beginning of validity= 2009:06:01	
Spatial Point = Point-2

Point-1		Position= 52°15′30″N/55°32′58″W	
Coordinate Reference System= WGS84	
Beginning of validity = 2017:01:01

Point-2		Position= 52°26′37″N/55°37′40″W	
Coordinate Reference System= WGS84	
Beginning of validity = 2009:06:01

=== End of File ===

## Example Territorial Sea

The following is an example simple dataset consisting of only the Territorial Sea Zone bounded by the Outer Limit of the Territorial Sea. The other boundaries of the Territorial Sea are described as Location By Text since they are not explicitly given. The landward limit is simply described as the coast. A minimum of lead-in metadata is included.

Title	Example Data Set

Language	English

Security Classification	Unclassified

Zone-1	Territorial Sea	Vertical Domain=	water_surface, water_column, seabed_surface, subsoil
Feature usage=	MLB
Releasability= official	
Beginning of validity= 1840	
Spatial Surface= Surface-1	
Plus Boundary= Limit-1, Limit-2

Surface-1		Coordinate Reference System= WGS84	
Beginning of validity = 1840:06
Surface Source=Source-1

Source-1	Spatial Source	Spatial source type = historical document	
Source name = Based on work by Sir John Franklin	
Source acceptance date = 1840:06:01	
Responsible party organization name = Library and Archive Canada	
Source online locator = http://www.archivescanada.ca/	
Beginning of validity=1931:12:11

Limit-1	Outer Limit of the Territorial Sea 	
Feature usage = MLB	
Feature context = Feature context = Calculated from historical survey	
Releasability= internal use	
Beginning of validity=1931:12:11	
Spatial Curve= Curve-1

Limit-2	Limit 	Feature usage = MLB	
Feature context= Generic closing limit 	
Releasability= internal use	
Spatial Curve= Curve-2

Curve-1		Line	lat1/long1, lat2/long2, lat3, long3, ….	
Coordinate Reference System= WGS84	
Beginning of validity = 1931:12:11

Curve-2		Location by text= closing line including the coastline and adjacent sides	
Beginning of validity = 1931:12:11

=== End of File ===

- The Carriage Return / Line Feed Delimiter

(Informative)

Record oriented communications systems have used the CR LF combination – Carriage Return CR (IA5 Code 13) followed by Line Feed LF (IA5 Code 10) since the earliest days of computing. The combination derives from the Model 33 Teletype machines that were used as the terminals on early computers. Since these terminals were physical printers there was need for time for the print carriage to return to the beginning of the line before it was possible to print the next line. The Teletype machine took up to 200 milliseconds to move the carriage back to the beginning of the line. At the print rate of 10 characters per second the combination CR followed by LF returned the carriage and moved to the next line within the time window.

All of the early computer systems adopted CR LF as the delimiter for a character string. Later when computer output was on video displayed some of the computer systems changed for efficiency. There is only a need for a single character to delimit the end of a character string or line. In those days storage space was precious.

Unfortunately all computer companies have not been consistent. Microsoft uses the CR LF combination – Carriage Return CR (IA5 Code 13) followed by Line Feed LF (IA5 Code 10). Apple used CR alone, and Unix (and recent Apple systems) use LF alone.  Most text processors accept either CR or LF or the combination CR LF.

This confusion remains to this day where some data is delimited with CR and other with LF. The CR LF combination seems to be read and interpreted by all systems. Therefore S-121 should use CR LF as a character string delimiter; however, one should expect that some display systems may print two blank lines between character strings rather than one. This does not affect the readability.

A second issue is that some attribute types allow unstructured text which may include the CR and / or LF character within the text. This needs to be allowed. A character string is not necessarily a record. An unstructured text string may contain several substrings (paragraphs) that may be terminated by a CR or LF or CR LF.

To address the need to have a unique end of record delimiter that has a clear printable visible effect that works in all environments a string or multiple characters has been selected.

The end of record delimiter would therefore be (IA5 Codes 09, 13, 10) TAB CR LF. The visual effect would be one (or occasionally 2) line spaces. The terminating TAB would not be seen.

- Abstract Syntax Notation

(Informative)

  - Introduction

The ISO 8824 Abstract Syntax Notation (ASN.1) is a meta language used to describe a data interchange format in a context-independent manner. ASN.1 itself is similar to the compiler description languages used in computer science to define programming languages. In fact the structure is similar to a programming language and a particular data file is analogous to a particular program written in the language. Like a language, the definition of a data file makes use of only those words in the language which apply in a given situation. Words, or statements composed of sequences of words, may be used as often as needed, in accordance with the syntactic rules of ASN.1 and the information model being encoded.

A number of basic data types make up the "vocabulary" of the "language". A complete description of the Abstract Syntax Notation would take some space to present. However in this document only a subset of the ASN commands need be used. This subset is identified below:

-  Assignment

-  CHOICE

-  SEQUENCE

-  SEQUENCE OF

-  SET

-  SET OF

-  MACRO

Quite complex data types can be described. This is accomplished by defining a small number of simple data types. The complete range of values of these simple data types may be defined, and then the manner of combining these simple data types into the more complex data types are specified. A data type defined in this manner may be assigned a tag so that it may be identified in the communication or otherwise distinguished. These tagged elements correspond to the classes in the UML model. Encoding rules described separately from the syntax of the data format define the manner in which the data types are encoded and delimited. There are several sets of encoding rules established by ISO for direct use with ASN.1. A set of binary encoding rules are described in ISO 8825; however, the syntax notation is not locked to any one particular set of encoding rules and any of the encoding rules described in the annexes of this document may be used to represent data corresponding to the syntax described.

  - Syntactic Structure

The Abstract Syntax Notation defines a communications format in terms of a syntactic tree. At the highest level the entire data set which is interchanged is described as a sequence of sub-sections. These sub-sections are then broken down into their component elements and so on, until each of the primitive data elements is identified. Each data element has a particular data type such as a basic text string, an integer etc. When data is communicated it is in terms of the syntactic tree. Many branches of the syntactic tree may be optional. When encoding using a tag oriented encoding technique such as ISO 8825 or XML the data stream is parsed by the receiving device and matched to the elements of the syntactic tree. In a tag oriented exchange formats, it is not necessary to pad the communications with null fields to accommodate information fields which are not used. The tag numbers associated with data elements allow the parser to skip over sections of the syntactic tree, so that only the relevant information need be communicated. This approach also allows for recursive definitions, so that there is no limit to the number or length of parameter data.

The S-121 Explicit Text Format is a tag oriented encoding. The tags are the Record Type Identifiers at the beginning of each record. Each record is terminated. In the S-121 Explicit Text Format each record is separate so there is no need for recursive structures.

    - Assignment

Branches of the syntactic tree are defined in terms of expressions in the Abstract Syntax Notation.  The Assignment operator  ::= equates a reference name to a series of more basic commands or primitive data elements. An entire syntax is described in terms of successive refinement. A data file consists of a number of named sections, each of which consists of a number of sub-sections etc. down to the level of the primitive data elements.

    - SEQUENCE and CHOICE

The basic operator in an assignment statement is the SEQUENCE command.  The SEQUENCE operator specifies that a branch of the syntactic tree consists of a defined number of elements in a fixed order; that is, a list of mandatory or optional elements. The SEQUENCE OF operator specifies that the number of elements is variable but that the order is fixed; that is, it indicates a repeated element or list of elements.  The SET operator specifies that a branch of the syntactic tree consists of a defined number of elements in any order. It is equivalent to a sequence but the order is not fixed. The SET OF operator specifies that the number of elements is variable and that the order is variable.  The CHOICE operator specifies that an instance within the syntactic tree permits the inclusion of one data element (or sub-branch) out of a fixed set of choices.

    - OPTIONAL and DEFAULT

Elements in a SEQUENCE or SET may be optional. This is, indicated by the keyword OPTIONAL immediately following the element name in the sequence or set.  If a default value is defined for an optional element then the keyword DEFAULT replaces the term OPTIONAL and the value of the default is specified following the DEFAULT keyword.

    - MACRO

A MACRO definition allows a primitive data type to be constructed out of other primitive data types. It also allows a default value notation to be defined for the constructed primitive so that the DEFAULT specification may be used with this new element.

    - Tag Numbers

Tag numbers are used to identify branches of the syntactic tree. A tag number is expressed in square brackets [ ] preceding an element name. Tag numbers are not necessary in a fixed SEQUENCE with no OPTIONAL elements since the order is known; however, in other cases, tag numbers must be supplied for each element. There are four classes of identifier tag codes: Universal, Context Specific, Application, and Private. Universal tags are used to identify operators such as CHOICE or SEQUENCE or basic data elements such as a GRAPHIC STRING.  Context Specific tags are assigned in each branch of the syntactic tree to number the elements of the tree in that branch. The same tag numbers are used over and over in different levels of the syntactic tree, and it is the responsibility of the syntax parser interpreting data encoded in terms of the ASN defined syntax to keep track of the levels of the tree and the local meaning of each tag. Application wide tags are used to define unique identifiers to specific elements. A parser does not need to keep track of the level of the syntax to identify the particular data element. Since Application wide tags are unique to a particular data element, long tag numbers would result if they were used extensively throughout a particular syntax. Therefore their use should be restricted to a few strategic places within the syntax. Private tags are used to build proprietary extensions to ASN.1, and are rarely used.

In certain situations redundant tags are generated. For example, a Universal tag may immediately follow a Context Specific tag. The keyword IMPLICIT is used to suppress the generation of a universal tag in those situations where the tag is unnecessary. An IMPLICIT SEQUENCE of two tagged elements would be encoded using only the context specific tags identifying each of the elements. The tag identifying the overall sequence would not be included.

    - Grammar

The following is a summary of the notations used in an example grammar.

::=  - is the production symbol of the grammar and can be read "is produced by" or "is composed of". A production allows the definition of a syntactic entity by the assignment of a name to a collection of data types or other entities. Entities placed on the right of the production symbol are taken together to define the composite entity.  By the use of production statements the elements of the grammar may be defined by successive refinement.

CHOICE {    }  - is the alternative indicator in a production definition. It can be read as "or". Alternate data entities are separated by commas within the brackets of the CHOICE indicator. Each of these entities must be identified by a TAG so that the particular entity communicated may be distinguished.  For example, the data entity Type consists of a choice of data element Type-1, Type-2, or Type-3. The TAG numbers are enclosed in square brackets.

Type    ::=  CHOICE   {     [1]     Type-1,

[2]     Type-2,

[3]     Type-3

}

SEQUENCE {   } - is the cumulative indicator in a production definition. It can be read as "and". A sequence of data entities are separated by commas within the brackets of the SEQUENCE indicator. For example, a data entity process consists of the ordered sequence of entities  Step-1, Step-2, Step-3.  Explicit TAGs are not required since the order of the sequence is fixed.

Process  ::=  SEQUENCE   {     Step-1,

Step-2,

Step-3

}

SET {    }   - is also a cumulative indicator in a production definition, which can also be read as "and". It is similar to the SEQUENCE indicator except that the order of the elements is not fixed. Therefore TAG codes must be used to distinguish the elements. For example, data entity Flock consists of the unordered sequence of entities Bird-1, Bird-2, Bird-3.

Flock    ::=   SET      {     [1]     Bird-1,

[2]     Bird-2,

[3]     Bird-3

}

SEQUENCE  OF  - is a series indicator in a production definition. It permits zero or more data entities of the same type to be part of a production. The end of the sequence is determined when there are no more data elements of the same type. For example, a data element List is composed of a series of Entries  (Entry-1, Entry-2, Entry-3,…).

List     ::=   SEQUENCE OF    Entries

Data entities in a SEQUENCE or a SET may be optional. Optional data elements are indicated by the keyword OPTIONAL after the data element name. A default value may also be defined for an optional data element. This is indicated by the keyword DEFAULT after the data element name followed by the value of the default state. The keyword DEFAULT implies the keyword OPTIONAL. For example, data entity Group consists of the ordered sequence of entities  Element-1, Element-2, Element-3, Element-4, where Element-2 is optional and Element-3 is optional and takes on the default value 1. Explicit TAGs are required in order to distinguish which elements are included.

Group   ::=  SEQUENCE   {  [1]  Element-1,

[2]  Element-2 OPTIONAL,

[3]  Element-3 DEFAULT (1),

[4]  Element-4

}

--    signifies the beginning of comments that are not part of the grammar, but are included to clarify the semantic meaning and context.

Keywords   - such as SEQUENCE, SET, CHOICE etc. are expressed in capital letters.

identifier name   - the name of an identifier consists of a string of letters, digits and hyphens beginning with a lower case letter. Identifier names may be used to identify particular elements of an entity or to assign a value to a particular element of an entity.

Entity Name    - the name of an entity consists of a string of letters, digits and hyphens beginning with an upper case letter.

The primitive level of the Abstract Syntax Notation consists of a number of built-in data types. These can be classified into several categories as shown below:

Numerical Data Types    - BOOLEAN          - True or False

INTEGER          - Integer number

(a signed integer number

of arbitrary length)

Bit Oriented Data Types - BIT STRING       - String of bits

OCTET STRING     - String of 8-bit bytes

General Data Types      - NULL             - Null string

ANY              - Any data string

Time/Date Data Type     - GENERALIZED TIME - Date / Time

Character Data Types    - GRAPHIC STRING    - A string of characters

These primitive data types correspond to those defined in ISO 19103 and used in the models in The ISO TC211 suite of standards.

This description of the Meta Notation only identifies the principal functions of ASN.1.  A more comprehensive description is given in the ISO standard ISO 8824.

- Implementation Schema

(Normative)

The Implementation Model for the S-121 standard is described in Appendix C.3 of the S-121 Product Specification document. It is reproduced here so that the various elements that need to be encoded can be identified.

Figure C1 – S-121 Implementation Model

The model in Figure C1 shows the major classes, but it also shows collection classes. The collection classes make the model look simpler but are not directly implemented. Also this model does not show the details of the S121_FeatureUnit and S121_SpatialAttributeType class.

The model in Figure C2 shows all of the implementation classes except for the details of the S121_FeatureUnit and S121_SpatialAttributeType which are shown separately. Otherwise the figure would be too complex to read. Note that there are many more relationships shown when only the instantiable classes are exposed.

In an implementation, relationships are addressed as pointers so they become a type of attribute. The composition relationship between S121_Party and S121_GroupParty includes a relationship class. This is effectively an attribute on an attribute and can be implemented as two attributes, one indicating participation of a party in a group and the other optionally indicating a percentage share in the participation.

Figure C2 – S-121 Instantiated Implementation Model

The structure of the S121_FeatureUnit and S121_SpatialAttributeType is shown in Figure C3. Since each Feature Unit and each Spatial Attribute Type can have a source there are a lot of connectors to the Source Group. Only the Zone Feature Unit and Space Feature Unit link to the Basic Administrative Unit S121_BAUnit.

Figure C3 – S-121 Instantiated Implementation Model for Feature and Attribute Group

Figure C4 shows the instantiable classes in the Feature/Attribute Group without the references to the Administrative Group or the Source Group.

Figure C4 – S-121 Instantiated Implementation Model for Feature and Attribute Group
 without External References

Each of the classes has a set of defined attributes. These are shown separately so that they can be linked to the encoding.

Also the navigable relations shown in the figures above (Figures C2, C3 and C4) will be implemented as pointers in the encoding and will therefore appear as attributes (containing the Oid of the object pointed to).

Figure C5 shows the Feature Unit. The Feature Unit will contain the name of the feature taken from Feature Catalogue. That is, it is a template for features such as “Territorial Sea”.  Note that all of the attributes are character strings or code list entries that can be put in character string fields. Some of the attributes, such as DateTime are structured text. A date would look like yyyy:mm:dd:hh:mm:ss:timezone for example 2017:04:16:13:23:14:-1. Parts of the date are optional but the delimiters are required.

Figure C5 – Implementation Class - S121_FeatureUnit

Figure C6 shows the Spatial Attribute Implementation classes.

Figure C6 – Implementation Class - S121_SpatialAttribute

Figures C7 shows the Location class. The geometry has reduced to as single Direct Position which is shown as a formatted character string.

Figure C7 – Implementation Class - S121_Location

Figures C8 shows the Limit class. The geometry has reduced to a structured PolyLine or Arc.  A vertex string is a structured character string. The Arc by Centre and Circle by Centre structures are also allowed since they are supported in S-100.

Figure C8 – Implementation Class - S121_Limit

Figures C9 shows the Zone class. The geometry is done by Zone to Limit relationships. The S121_Surface object carries attributes about the geometry but not the actual geometry.

Figure C9 – Implementation Class  - S121_Zone

Figures C10 shows the Space class. Space is essentially a zone with a reference point and a height. The position is a direct position and a vertical.

Figure C10 – Implementation Class  - S121_Space

Figure C11 shows the implementation of the Basic Administrative Unit class. This is a simple class with only a few character string attributes.

Figure C11 – Implementation Class  - S121_BAUnit

Each of the Rights, Restrictions and Responsibilities classes are treated separately. This is shown in Figures C12, C13 and C14. The MA_RRRshare attribute is shown as a Fraction data type and MA_RRRshareCheck is shown as Boolean. The fraction and Boolean also need to be represented as character string elements.

Figure C12 – Implementation Class  - S121_Right

Figure C13 – Implementation Class  - S121_ Restriction

Figure C14 – Implementation Class  - S121_ Responsibility

Figures C15 shows the S121 PartyGroup classes. The composition relationship between S121_Party and S121_GroupParty can be implemented as a pointer (or set of pointers). The optional S121_PartyMember MA_partyShare attribute can be implemented as one to three attributes following the pointer.

Figure C15 – Implementation Class  - S121_ PartyGroup

Figure C16 shows the implementation of the class Spatial Source. This complex structure has been flattened to a single class with a lot of attributes, most of which are optional.

Figure C16 – Implementation Class  - S121_SpatialSource

Figure C17 shows the implementation of the class Administrative Source. This is similar to Spatial Source and the structure has been flattened to a single class with a many optional character string attributes.

Figure C17 – Implementation Class  - S121_AdministrativeSource

- Encoding Tables

(Normative)

The names of features and attributes in the Application Schema model are UML names that are unique within the entire context of the IHO and ISO TC211 suite of standards. They are cryptic to not be too long and also contain extra lead-in characters to ensure uniqueness. They are appropriate for use in a UML model or an XML schema but are not appropriate for use in a human readable explicit text format.

In a human readable explicit text format the object, feature and attribute names need to be clear and readable text.

The object types in identified records are uniquely identified by the name space names used as the beginning portion of the record numbers. These are:

| - Record Type | - Namespace identifier (used as the lead-in to the Record Number) |
| --- | --- |
| - Location Feature Type Record | - Location- |
| - Limit Feature Type Record | - Limit- |
| - Zone Feature Type Record | - Zone- |
| - Space Feature Type Record | - Space- |
| - Point Spatial Attribute Record | - Point- |
| - Curve Spatial Attribute Record | - Curve- |
| - Surface Spatial Attribute Record | - Surface- |
| - Basic Administrative Unit Information Object Record | - Administrative Unit- |
| - Right Information Object Record | - Right- |
| - Restriction Information Object Record | - Restriction- |
| - Responsibility Information Object Record | - Responsibility- |
| - Party Group Information Object Record | - Party- |
| - Spatial Source Information Object Record | - Source- |
| - Administrative Source Information Object Record | - Source- |

Table D1 – Identifiable Record Type Encoding

The special records are also uniquely identified by the Record Type Identifier. Note that since the Record Type Identifier and the Record Number Namespace Identifier both are the beginning entities of records they need to be non-interfering lists.

| - Record Type | - Namespace identifier (used as the lead-in to the Record Number) |
| --- | --- |
| - Metadata - Title | - Title |
| - Metadata - S-100 version | - S-100 version |
| - Metadata - S121 version | - S121 version |
| - Metadata - S121 version date | - S121 version date |
| - Metadata - S121 language | - S121 language |
| - Metadata - Security classification | - Security classification |
| - Metadata - S121 contact | - S121 contact |
| - Metadata - S121 specification URL | - S121 specification URL |
| - Metadata - S121 identifier (“S-121”) | - S121 identifier |
| - Metadata - S121 maintenance | - S121 maintenance |
| - End of File Record | - === End of File === |

Table D2 – Special Record Type Encoding

The attribute identifiers used in the UML application schema must be translated into readable text. Table D2 provides a bidirectional translation table between Attribute identifier and Readable name. The other columns in the table: Data type, Obligation and Definition are informative and provided for the assistance of a developer. This information is already available in the UML model.

| Attribute identifier | Readable name | Data type | Obligation | Definition |
| --- | --- | --- | --- | --- |
| MA_TypeName | Feature name | Character String | 0..1 | The name of the feature is optionally defined in the attribute MA_TypeName. |
| MA_Type | Feature usage | Codelist Character String |  | The attribute MA_Type describes the type of the feature. Its value is a code from the code list S121_FeatureType. This code list includes types that have a common characteristic related to the marine environment. The code list is registered in the Feature Concept Dictionary as listed values and as such can be expanded to include all aspects of the legal context. The initial contents are: MLB (Marine Limits and Boundaries) and can be extended to A76 (UNCLOS article 76) and other. |
| MA_Label | Feature code | Character String | 0..1 | The attribute MA_Label provides a short textual identifier of the feature unit. |
| MA_Context | Feature context | Character String | 0..1 | The attribute MA_Context optionally describes the legal or administrative aspects of the feature object. |
| MA_Releasability | Releasability | Character String | 0..1 | The attribute MA_Releasability is optionally used to differentiate between "official", "development", "internal use", or "in construction" or other status for particular features. This attribute is a Character String because the status of a feature is dependent upon the procedures used within the organization the developed the feature instance. |
| beginLifespanVersion | Beginning of validity | Structured Character String |  | Start time of a specific instance version. |
| endLifespanVersion | End of validity | Structured Character String | 0..1 | End time of a specific instance version. |
| MA_locationByText | Location by text | Character String | 0..1 | The attribute MA_locationByText allows a spatial attribute to be a textual description |
| MA_referenceSystem | Coordinate Reference System | Structured Character String | 0..1 | The attribute MA_referenceSystem allows a CoordinateReferencingSystem (CRS) to optionally be specified at the S121_SpatialAttributeType level. |
| scaleMaximum | Maximum scale | Character String | 0..1 | Maximum scale at which the data can be used |
| scaleMinimum | Minimum scale | Character String | 0..1 | Minimum scale at which the data can be used |
| MA_orgtype | Computational origin | Codelist Character String | 0..1 | Computational origin of the element: defined, densified, computed or construction |
| MA_interpolationRole | Interpolation role | Codelist Character String |  | The role of point in the structure of a straight line or curve |
| MA_pointType | Type of point | Codelist Character String | 0..1 | The type of point |
| MA_TransformedLocation | Adjusted location | Formatted Character String | 0..1 | Location of transformed point |
| MA_arctyp | Type of arc | Codelist Character String | 0..1 | Type of computation used to define an arc (line) geodesic or loxodrome |
| MA_verdom | Vertical domain | Codelist Character String | 0..* | Category of vertical domain of the object: airspace, land_surface, water_surface, water_column, seabed_surface, subsoil. Any particular object may span more than one vertical domain. |
| MA_areaSize | Area size | Character String | 0..* | The area value |
| MA_areaType | Area type | Codelist Character String | 0..* | Area type: official area, non-official area, calculated area, surveyed area |
| MA_referencePoint | Reference point | Formatted Character String | 0..1 | The coordinates of a point inside the spatial unit |
| MA_surfaceRelation | Surface relationship | Codelist Character String | 0..1 | Relationship to surface |
| MA_UnitOfMeasure | Unit of measure | Character String | 0..1 | Unit of measure |
| MA_VolumeSize | Volume size | Character String | 0..* | The volume of the space |
| MA_VolumeType | Volume Type | Codelist Character String | 0..1 | The type of volume |
| MA_PointGeometry | Position | Formatted Character String |  | GM_Point geometry as per IHO S100 V2 Part 7 Spatial Schema |
| MA_CurveGeometry | Line | Formatted Character String |  | GM_OrientableCurve geometry as per IHO S100 V2 Part 7 Spatial Schema. Spatial primitives limited to GM_Curve, and GM_Curve (arcByCentrePoint and circleByCentrePoint) as per S-100. |
| MA_verticalPosition | Vertical reference level | Formatted Character String |  | Start level for vertical extent |
| MA_verticalExtent | Vertical Extent | Character String |  | Vertical extent of space |
| MA_BAname | Administrative unit type | Character String | 0..1 | MA_BAname optionally establishes name for an instance of a BAUnit. |
| MA_BAtype | Administrative unit name | Codelist Character String |  | The attribute MA_BAtype describes the use type of a BAUnit |
| MA_BAcontext | Administrative unit context | Character String | 0..1 | The attribute MA_BAcontext allows the context for an instance of a BAUnit to be described. |
| MA_RRRdescription | Rights Restrictions Responsibility description | Character String | 0..1 | Description regarding the right, restriction or responsibility |
| MA_RRRshare | Share | Character String | 0..1 | A share in an instance of a subclass of LA_RRR |
| MA_RRRshareCheck | Share constraint applicable | Structured Character String | 0..1 | Boolean indicating whether the constraint is applicable |
| MA_RRRtimeSpec | Share of a right in time | Structured Character String | 0..1 | Operational use of a right in time sharing |
| MA_RightType | Right type | Codelist Character String |  | The type of the right |
| MA_RestrictionType | Restriction type | Codelist Character String |  | The type of the restriction |
| MA_partyRequired | Party Required | Structured Character String | 0..1 | Boolean indicating whether a party is required for the registration of the restriction in the association to LA_Party |
| MA_ResponsibilityType | Responsibility type | Codelist Character String |  | The type of the responsibility |
| extPID | External party identifier | Formatted Character String | 0..1 | The identifier of the party in an external registration |
| MA_partyName | Party name | Character String | 0..1 | The name of the party |
| MA_partyRole | Party role | Character String | 0..* | The role of the party |
| MA_partyType | Party type | Codelist Character String |  | The type of the party |
| MA_partyShare | Group party member share | Character String | 0..1 | The fraction of the whole for a party member |
| MA_groupPartyType | Group party type | Codelist Character String |  | The type of the group party |
| MA_name | Source name | Character String | 0..1 | Document name - for example the document (legislation, treaty, title) that defines the object |
| MA_SourceORlinkageURL | Source online locator | Character String | 0..* | Location for online access |
| MA_SourceORprotocol | Source online protocol | Character String | 0..* | Online Resource Protocol |
| MA_SourceORapplicationProfile | Source application profile | Character String | 0..* | Name of an application profile that can be used with the resource |
| MA_SourceORname | Source name | Character String | 0..* | Name of the resource |
| MA_SourceORdescription | Source description | Character String | 0..* | Description of what the resource is/does |
| MA_SourceORfunction | Source function | Codelist Character String | 0..* | Function performed by the resource: download, information, offline access, order, search |
| MA_RegistryNumber | Source registry number | Character String | 0..1 | Unique official identifier of the record in a registry. For example, in states with registers of legislative instruments, versioning is controlled by the registry ID |
| MA_lifeSpanStamp | Source processed date | Structured Character String | 0..1 | The moment that the event represented by the instance of LA_Source is further processed in the LA system |
| MA_acceptance | Source acceptance date | Structured Character String | 0..1 | The date of force of law of the source by an authority |
| MA_maintype | Source physical type | Codelist Character String | 0..1 | the physical type of document |
| MA_availabilityStatus | Source availability status | Codelist Character String |  | The availability status of document |
| MA_submission | Source submission date | Structured Character String | 0..1 | The date of submission of the source by a party |
| MA_recordation | Source date of recordation | Structured Character String | 0..1 | The date of registration (recordation) of the source by registering authority |
| MA_RPindividualName | Responsible party name | Character String | 0..1 | Name of the responsible individual |
| MA_RPcontactORlinkageURL | Responsible party online contact | Character String | 0..1 | Responsible party location for online access |
| MA_RPcontactORprotocol | Responsible party online protocol | Character String | 0..1 | Responsible party connection protocol to be used |
| MA_RPcontactORapplicationProfile | Responsible party application profile | Character String | 0..1 | Name of an application profile that can be used with the resource |
| MA_RPcontactORname | Responsible party resource description | Character String | 0..1 | Name of resource |
| MA_RPcontactORdescription | Responsible party resource name | Character String | 0..1 | Description of what the resource is/does |
| MA_RPcontactORfunction | Responsible party resource function | Codelist Character String | 0..1 | Function performed by the resource: download, information, offline access, order, search |
| MA_RPorganizationName | Responsible party organization name | Character String | 0..1 | Name of the responsible organization |
| MA_RPpositionName | Responsible party position | Character String | 0..1 | Position of the responsible person |
| MA_RPcontactPhone | Responsible party contact phone | Character String | 0..1 | Telephone numbers at which the organisation or individual may be contacted |
| MA_RProle | Responsible party role | Codelist Character String | 0..1 | Function performed by the responsible party: resource provider, custodian, owner, user, distributor, originator, point of contact, principle investigator, processor, publisher, author |
| MA_RPcontactAddressCountry | Responsible party address country | Character String | 0..1 | Country of the physical address |
| MA_RPcontactAddressDeliveryPoint | Responsible party delivery point | Character String | 0..1 | Address line for the physical address (Street name, box number, suite) |
| MA_RPcontactAddressCity | Responsible party city | Character String | 0..1 | City of the physical address |
| MA_RPcontactElectronicMailAddress | Responsible party electronic mail address | Character String | 0..1 | Electronic Mail address |
| MA_RPcontactAddressAdministrativeArea | Responsible party state/province | Character String | 0..1 | State, province of the physical address |
| MA_RPcontactAddressPostalCode | Responsible party postal code | Character String | 0..1 | Postal Code for the physical address |
| DQnameOfMeasure | Data Quality name of measure | Character String | 0..1 | Data Quality name of measure |
| DQmeasureIdentification | Data Quality measure identification | Character String | 0..1 | Data Quality measure identification |
| DQmeasureDescription | Data Quality measure description | Character String | 0..1 | Data Quality measure description |
| DQevaluationMethodType | Data Quality evaluation method type | Codelist Character String | 0..1 | Data Quality evaluation method type |
| DQevaluationMethodDescription | Data Quality evaluation method description | Character String | 0..1 | Data Quality evaluation method description |
| DQevaluationProcedure | Data Quality evaluation method type | Character String | 0..1 | Data Quality evaluation method type |
| DQdateTime | Data Quality reference date | Structured Character String | 0..1 | Data Quality reference date |
| DQresult | Data Quality result | Character String | 0..2 | Data Quality result |
| MA_ExternalArchiveAcceptance | External Archive date of acceptance | Structured Character String | 0..1 | The date of force of law of the source by the authority |
| MA_ExternalArchiveData | External Archive information | Character String | 0..1 | The content of the source |
| MA_ExternalArchiveExtraction | External Archive date of retrieval | Structured Character String | 0..1 | Date of retrieval of information from an external archive |
| MA_ExternalArchiveRecordation | External Archive date of recordation | Structured Character String | 0..1 | Date information was recorded in an external archive |
| MA_ExternalArchive_sID | External Archive Identifier | Formatted Character String | 0..1 | Identifier of the external archive |
| MA_ExternalArchiveSubmission | External Archive date of submission | Structured Character String | 0..1 | Date of submission of information in an external archive by a party |
| MA_SpatialSourceType | Spatial source type | Character String | 0..1 | The type of spatial source document |
| MA_AdminSourceType | Administrative source type | Codelist Character String | 0..1 | The type of administrative source document |

Table D3 – Attribute Identifier Encoding

- Record Oriented Encoding Structure
 in Abstract Syntax Notation ASN.1

(Normative)

This Appendix describes the Explicit Text Record Oriented Format using the Abstract Syntax Notation (ASN.1). A description of the ASN.1 description language is given in Appendix B. This syntax description uses the same attribute type names as used in the Implementation Model given in Appendix C. It also uses the readable text names as described in Appendix D.

The metadata elements are derived from the S-121 Product Specification that themselves are derived from S-100. Additional metadata may be added in accordance with S-100 and ISO 19115 Geographic Information Metadata..

S-121 Data Set Abstract Syntax

S121DataSet

::= SEQUENCE {

S121MetadataModule,	-- Discovery metadata

SEQUENCE OF

S121_IdentifiedRecord, 	-- Multiple MLB_Record items 
-- may exist in a data set
-- in accordance with the
-- S121 Application Schema.

S121_EndOfFileRecord	-- The file is terminated with 
-- an explicit human readable 
-- End of File Record.

}

-- Each S121_dataset begins with a metadata header as specified in the Product

-- Specification. The metadata elements that are required are defined by S-100.

S121MetadataModule

::= SEQUENCE {

[1] S121_Title,	-- Title of document

[2] S100_Version DEFAULT (“2.0”),	-- Version of S-100 on which S-121 Product Specification is based.

[3] S121_Version DEFAULT (“1.0”),	-- Version of S-121 Product Specification

[4] S121_VersionDate OPTIONAL,	-- Date the product specification was created / last updated

[5] SEQUENCE OF {S121_Language},	-- Language(s) of the data product specification,

[6] S121_SecurityClassification DEFAULT (“unclassified”),
-- Security classification code on the data product specification

[7] S121_Contact,	-- Party responsible for the data product specification

[8] S121_SpecificationURL OPTIONAL,	-- Online-address where the resource is downloadable

[7] S121_Identifier DEFAULT (“S-121”),	-- Persistent unique identifier for a published version of the product specification.

[9] S121_Maintenance,	-- Description of the maintenance regime for data set.

}

S121_Title 	::= Graphic String

S100_Version	::= Graphic String

S121_Version	::= Graphic String

S121_VersionDate 	::= Graphic String

S121_Language	::= Graphic String

S121_SecurityClassification 	::= Graphic String

S121_Contact	::= Graphic String

S121_SpecificationURL 	::= Graphic String

S121_Identifier 	::= Graphic String

S121_Maintenance,	::= Graphic String

S121_EndOfFileRecord 	::= Graphic String {”=== End of File ===”}

-- Each S121_IdentifiedRecord consists of a S121_RecordNumber followed by a

-- S121_RecordTypeIdentifier followed by a S121_RecordContents and terminated by an

-- EndOfRecordMarker. In the Explicit Text Encoding these fields are separated

-- by a TAB delimiter and the EndOfRecordMarker is the sequence of characters CR LF.

S121_IdentifiedRecord

::= CHOICE {

S121_LocationRecord,	-- Location Feature Type Record

S121_LimitRecord,	-- Limit Feature Type Record

S121_ZoneRecord,	-- Zone Feature Type Record

S121_SpaceRecord,	-- Space Feature Type Record

S121_PointRecord,	-- Point Feature Type Record

S121_CurveRecord,	-- Curve Feature Type Record

S121_BAUnitRecord,	-- Basic Administrative Unit Feature Type Record

S121_RightRecord,	-- Right Feature Type Record

S121_RestrictionRecord,	-- Restriction Feature Type Record

S121_ResponsibilityRecord,	-- Responsibility Feature Type Record

S121_PartyRecord,	-- Party Feature Type Record

S121_SourceRecord,	-- Spatial Source or Administrative Source Feature Type Record

}

RightNamespace 	::= Graphic String	{”Right-”}

RestrictionNamespace 	::= Graphic String	{”Restriction-”}

ResponsibilityNamespace 	::= Graphic String	{”Responsibility-”}

PartyNamespace 	::= Graphic String	{”Party-”}

SourceNamespace 	::= Graphic String	{”Source-”}

RecordNumbNumericID 	::= Graphic String	-- Unique number in the namespace for the record instance. In the explicit Text Format this is represented as a character string.

--    -------------------------------------------------------------------------------

S121__LocationRecord

::= SEQUENCE {

SEQUENCE	{LocationRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

LocationTypeIdentifier,	-- Unique identifier of the type of record

Location_RecordContent,	-- Set of attributes pertaining to the specific record type

}

LocationNamespace 	::= Graphic String	{”Location-”}

LocationTypeIdentifier 	::= Graphic String	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

LocationRecordContent

::= SEQUENCE {

[1] MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

[2] MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

[3] MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

[4] MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

[5] MA_orgtype OPTIONAL,	-- Computational origin of the element: defined, densified, computed or construction

[6] MA_interpolationRole,	-- Computational origin of the element: defined, densified, computed or construction

[7] MA_pointType OPTIONAL,	-- The type of point

[8] TransformedLocation OPTIONAL,	-- Location of transformed point

[9] beginLifespanVersion,	-- Start time of a specific instance version

[10] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_LimitRecord

::= SEQUENCE {

SEQUENCE	{LimitRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

LimitTypeIdentifier,	-- Unique identifier of the type of record

Limit_RecordContent,	-- Set of attributes pertaining to the specific record type

}

LimitNamespace 	::= Graphic String	{” Limit-”}

LimitTypeIdentifier 	::= Graphic String	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Limit type in the Feature Catalogue..

LimitRecordContent

::= SEQUENCE {

[1] MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

[2] MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

[3] MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

[4] MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

[5] MA_arctyp OPTIONAL,	-- Type of computation used to define an arc (line) geodesic or loxodrome

[6] beginLifespanVersion,	-- Start time of a specific instance version

[7] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_ZoneRecord

::= SEQUENCE {

SEQUENCE	{ZoneRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

ZoneTypeIdentifier,	-- Unique identifier of the type of record

Zone_RecordContent,	-- Set of attributes pertaining to the specific record type

}

ZoneNamespace 	::= Graphic String	{” Zone-”}

ZoneTypeIdentifier 	::= Graphic String	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Zone type in the Feature Catalogue.

ZoneRecordContent

::= SEQUENCE {

[1] MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

[2] MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

[3] MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

[4] MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

[5] MA_verdom OPTIONAL,	-- Category of vertical domain of the object delimited.

[6] SEQUENCE OF {MA_areaSize} OPTIONAL,	-- The area value is represented as a character string value in a Unit of Measure.

[7] MA_areaType OPTIONAL,	-- Area type: official area, non-official area, calculated area, surveyed area

[8] MA_referencePoint OPTIONAL,	-- The coordinates of a point inside the spatial unit

[9] MA_surfaceRelation OPTIONAL,	-- Relationship to surface

[10] MA_UnitOfMeasure OPTIONAL,	-- Units used to describe area size

[11] beginLifespanVersion,	-- Start time of a specific instance version

[12] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_SpaceRecord

::= SEQUENCE {

SEQUENCE	{SpaceRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

SpaceTypeIdentifier,	-- Unique identifier of the type of record

Space_RecordContent,	-- Set of attributes pertaining to the specific record type

}

SpaceNamespace 	::= Graphic String	{” Space-”}

SpaceTypeIdentifier 	::= Graphic String	-- Type of record as described from the generic Space type in the Feature Catalogue.

SpaceRecordContent

::= SEQUENCE {

[1] MA_Type OPTIONAL,	-- Describes the type of the feature from the code list S121_FeatureType.

[2] MA_Label OPTIONAL,	-- A short textual identifier of the feature unit (e.g. a code)

[3] MA_Context OPTIONAL,	-- The legal or administrative aspects of the feature object

[4] MA_Releasability OPTIONAL,	-- Releasability status of the feature object (official, development, internal use, in construction or other).

[5] MA_verdom OPTIONAL,	-- Category of vertical domain of the object delimited.

[6] MA_referencePoint OPTIONAL,	-- The coordinates of a point inside the spatial unit

[7] SEQUENCE OF {MA_volumeSize} OPTIONAL,	-- The volume size value is represented as a character string value in a Unit of Measure.

[8] MA_UnitOfMeasure OPTIONAL,	-- Units used to describe volume size

[9] MA_volumeType OPTIONAL,	-- Volume type: official volume, non-official volume, calculated volume, surveyed volume

[10] beginLifespanVersion,	-- Start time of a specific instance version

[11] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_PointRecord

::= SEQUENCE {

SEQUENCE	{PointRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

Point_RecordContent,	-- Set of attributes pertaining to the specific record type

}

-- Since there is only one type of Point Record the Type field is not used in the Point Record

PointNamespace 	::= Graphic String	{” Point-”}

.

PointRecordContent

::= SEQUENCE {

[1] MA_PointGeometry,	-- Lat / Long of a Direct Position.

[2] MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

[3] MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

[4] scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

[5] scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

[6] beginLifespanVersion,	-- Start time of a specific instance version

[7] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_CurveRecord

::= SEQUENCE {

SEQUENCE	{CurveRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

Curve_RecordContent,	-- Set of attributes pertaining to the specific record type

}

-- Since there is only one type of Curve Record the Type field is not used in the Curve Record

CurveNamespace 	::= Graphic String	{” Curve-”}

.

CurveRecordContent

::= SEQUENCE {

[1] MA_CurveGeometry,	-- A set of vertices( Lat / Long Direct Position) or an Arc by Centre Point or a Circle By Centre Point .

[2] MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

[3] MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

[4] scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

[5] scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

[6] beginLifespanVersion,	-- Start time of a specific instance version

[7] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_SurfaceRecord

::= SEQUENCE {

SEQUENCE	{SurfaceRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

Surface_RecordContent,	-- Set of attributes pertaining to the specific record type

}

-- Since there is only one type of Surface Record the Type field is not used in the Surface Record

SurfaceNamespace 	::= Graphic String	{” Surface-”}

.

SurfaceRecordContent

::= SEQUENCE {

[1] MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

[2] MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

[3] scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

[4] scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

[5] beginLifespanVersion,	-- Start time of a specific instance version

[6] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121_VolumeRecord

::= SEQUENCE {

SEQUENCE	{VolumeRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

Volume_RecordContent,	-- Set of attributes pertaining to the specific record type

}

-- Since there is only one type of Volume Record the Type field is not used in the Volume Record

SurfaceNamespace 	::= Graphic String	{” Volume-”}

.

VolumeRecordContent

::= SEQUENCE {

[1] MA_verticalPosition,	-- Start level for vertical extent

[2] MA_verticalExtent,	-- Describes Vertical extent of space

[3] MA_UnitOfMeasure OPTIONAL,	-- Describes Unit of measure of the vertical extent

[4] MA_locationByText OPTIONAL,	-- Describes the location of the feature as a textual statement

[5] MA_ReferenceSystem OPTIONAL,	-- Allows a Coordinate Reference System (CRS) to optionally be specified

[6] scaleMaximum OPTIONAL,	-- Maximum scale at which the data can be used.

[7] scaleMinimum OPTIONAL,	-- Minimum scale at which the data can be used.

[8] beginLifespanVersion,	-- Start time of a specific instance version

[9] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__BAUnitRecord

::= SEQUENCE {

SEQUENCE	{BAUnitRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

BAUnitTypeIdentifier,	-- Unique identifier of the type of record

BAUnit_RecordContent,	-- Set of attributes pertaining to the specific record type

}

BAUnitNamespace 	::= Graphic String	{”Unit-”}

BAUnitTypeIdentifier 	::= Graphic String	{”Administrative Unit”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

BAUnitRecordContent

::= SEQUENCE {

[1] MA_BAtype,	-- the use type of a BAUnit: SovereigntyUnit, SoverignRightsUnit, JointDevelopmentUnit, OtherRegulatoryUnit

[2] MA_Context OPTIONAL,	-- allows the context for an instance of a BAUnit to be described.

[3] beginLifespanVersion,	-- Start time of a specific instance version

[4] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__RightRecord

::= SEQUENCE {

SEQUENCE	{RightRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

RightTypeIdentifier,	-- Unique identifier of the type of record

Right_RecordContent,	-- Set of attributes pertaining to the specific record type

}

RightNamespace 	::= Graphic String	{”Right-”}

RightTypeIdentifier 	::= Graphic String	{”Administrative Right”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

RightRecordContent

::= SEQUENCE {

[1] MA_RightType,	-- the type of the right: sovereignty, sovereignRight, accessRight, harvestRight, easementRight

[2] MA_RRRdescription OPTIONAL,	-- description regarding the right.

[3] MA_share OPTIONAL,	-- share of an instance of a right

[4] MA_shareCheck OPTIONAL,	-- boolean indicating whether the constraint is applicable

[5] MA_timeSpec OPTIONAL,	-- operational use of a right in time sharing.

[6] beginLifespanVersion,	-- Start time of a specific instance version

[7] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__RestrictionRecord

::= SEQUENCE {

SEQUENCE	{RestrictionRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

RestrictionTypeIdentifier,	-- Unique identifier of the type of record

Restriction_RecordContent,	-- Set of attributes pertaining to the specific record type

}

RestrictionNamespace 	::= Graphic String	{”Restriction-”}

RestrictionTypeIdentifier 	::= Graphic String	{”Administrative Restriction”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

RestrictionRecordContent

::= SEQUENCE {

[1] MA_partyRequired,	-- indicates whether a party is required for the registration of the restriction in the association to Party

[2] MA_RestrictionType,	-- the type of the Restriction: timeBasedRestriction, passageRestriction, accessRestriction, useRestriction, jurisdictionRestriction, resourceRestriction

[3] MA_RRRdescription OPTIONAL,	-- description regarding the right, restriction or responsibility.

[4] MA_share OPTIONAL,	-- share of an instance of a right

[5] MA_shareCheck OPTIONAL,	-- boolean indicating whether the constraint is applicable

[6] MA_timeSpec OPTIONAL,	-- operational use of a right in time sharing.

[7] beginLifespanVersion,	-- Start time of a specific instance version

[8] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__ResponsibilityRecord

::= SEQUENCE {

SEQUENCE	{ResponsibilityRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

ResponsibilityTypeIdentifier,	-- Unique identifier of the type of record

Responsibility_RecordContent,	-- Set of attributes pertaining to the specific record type

}

ResponsibilityNamespace 	::= Graphic String	{”Responsibility-”}

ResponsibilityTypeIdentifier 	::= Graphic String	{”Administrative Responsibility”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

ResponsibilityRecordContent

::= SEQUENCE {

[1] MA_ResponsibilityType,	-- the type of the responsibility: maintenanceResponsibility

[2] MA_RRRdescription OPTIONAL,	-- description regarding the right, restriction or responsibility.

[2] MA_share OPTIONAL,	-- share of an instance of a right

[2] MA_shareCheck OPTIONAL,	-- boolean indicating whether the constraint is applicable

[2] MA_timeSpec OPTIONAL,	-- operational use of a right in time sharing.

[3] beginLifespanVersion,	-- Start time of a specific instance version

[4] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__PartyRecord

::= SEQUENCE {

SEQUENCE	{PartyRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

PartyTypeIdentifier,	-- Unique identifier of the type of record

Party_RecordContent,	-- Set of attributes pertaining to the specific record type

}

PartyNamespace 	::= Graphic String	{”Party-”}

PartyTypeIdentifier 	::= Graphic String	{”Party”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

PartyRecordContent

::= SEQUENCE {

[1] exPID,	-- the identifier of the party in an external registration

[2] SEQUENCE OF {MA_PartyRole} OPTIONAL,	-- the role of the party

[3] MA_partyType,	-- the type of the party

[4] beginLifespanVersion,	-- Start time of a specific instance version

[5] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__GroupPartyRecord

::= SEQUENCE {

SEQUENCE	{GroupPartyRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

GroupPartyTypeIdentifier,	-- Unique identifier of the type of record

GroupParty_RecordContent,	-- Set of attributes pertaining to the specific record type

}

GroupPartyNamespace 	::= Graphic String	{”Party-”}

GroupPartyTypeIdentifier 	::= Graphic String	{”GroupParty”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

GroupPartyRecordContent

::= SEQUENCE {

[1] MA_groupPartyType,	-- the type of the group party: agreement, association

[2] exPID,	-- the identifier of the party in an external registration

[3] SEQUENCE OF {MA_PartyRole} OPTIONAL,	-- the role of the party

[4] MA_partyType,	-- the type of the party

[5] beginLifespanVersion,	-- Start time of a specific instance version

[6] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__SpatialSourceRecord

::= SEQUENCE {

SEQUENCE	{SpatialSourceRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

SpatialSourceTypeIdentifier,	-- Unique identifier of the type of record

SpatialSourceRecordContent,	-- Set of attributes pertaining to the specific record type

}

SpatialSourceNamespace 	::= Graphic String	{”Source-”}

SpatialSourceTypeIdentifier 	::= Graphic String	{”Spatial Source”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

SpatialSourceContent

::= SEQUENCE {

[1] MA_SpatialSourceType OPTIONAL,	-- The type of spatial source document

[2] SEQUENCE OF {MA_SourceORlinkageURL} OPTIONAL,	-- Location for online access

[3] SEQUENCE OF {MA_SourceORprotocol OPTIONAL,	-- Online Resource Protocol

[4] SEQUENCE OF {MA_SourceORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

[5] MA_SourceORname OPTIONAL,	-- Name of the resource

[6] MA_SourceORdescription OPTIONAL,	-- Description of what the resource is/does

[7] MA_SourceORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

[8] MA_RegistryNumber OPTIONAL,	-- Unique official identifier of the record in a registry

[9] MA_lifeSpanStamp OPTIONAL,	-- The moment that the event represented by the instance of LA_Source is further processed in the LA system

[10] MA_acceptance OPTIONAL,	-- The date of force of law of the source by an authority

[11] MA_maintype OPTIONAL,	-- The physical type of document

[12] MA_availabilityStatus OPTIONAL,	-- The availability status of document

[13] MA_submission OPTIONAL,	-- The date of submission of the source by a party

[14] MA_recordation OPTIONAL,	-- The date of registration (recordation) of the source by registering authority

[15] MA_RPindividualName OPTIONAL,	-- Name of the responsible individual

[16] MA_RPcontactORlinkageURL OPTIONAL,	-- Responsible party location for online access

[17] MA_RPcontactORprotocol OPTIONAL,	-- Responsible party connection protocol to be used

[18] MA_RPcontactORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

[19] MA_RPcontactORname OPTIONAL,	-- Name of resource

[20] MA_RPcontactORdescription OPTIONAL,	-- Description of what the resource is/does

[21] MA_RPcontactORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

[22] MA_RPorganizationName OPTIONAL,	-- Name of the responsible organization

[23] MA_RPpositionName OPTIONAL,	-- Position of the responsible person

[24] MA_RPcontactPhone OPTIONAL,	-- Telephone numbers at which the organisation or individual may be contacted

[25] MA_RProle OPTIONAL,	-- Function performed by the responsible party

[26] MA_RPcontactAddressCountry OPTIONAL,	-- Country of the physical address

[27] MA_RPcontactAddressDeliveryPoint OPTIONAL,	-- Address line for the physical address (Street name, box number, suite)

[28] MA_RPcontactAddressCity OPTIONAL,	-- City of the physical address

[29] MA_RPcontactElectronicMailAddress OPTIONAL,	-- Electronic Mail address

[30] MA_RPcontactAddressAdministrativeArea OPTIONAL,	-- State, province of the physical address

[31] MA_RPcontactAddressPostalCode OPTIONAL,	-- Postal Code for the physical address

[32] SEQUENCE OF{DQnameOfMeasure} OPTIONAL,	-- Data Quality name of measure

[33] DQmeasureIdentification OPTIONAL,	-- Data Quality measure identification

[34] DQmeasureDescription OPTIONAL,	-- Data Quality measure description

[35] DQevaluationMethodType OPTIONAL,	-- Data Quality evaluation method type

[36] DQevaluationMethodDescription OPTIONAL,	-- Data Quality evaluation method description

[37] DQevaluationProcedure OPTIONAL,	-- Data Quality evaluation method type

[38] SEQUENCE OF {DQdateTime} OPTIONAL,	-- Data Quality reference date

[39] SEQUENCE OF {DQresult},	-- Data Quality result

[40] MA_ExternalArchiveAcceptance OPTIONAL,	-- The date of force of law of the source by the authority

[41] MA_ExternalArchiveData OPTIONAL,	-- The content of the source

[42] MA_ExternalArchiveExtraction OPTIONAL,	-- Date of retrieval of information from an external archive

[43] MA_ExternalArchiveRecordation OPTIONAL,	-- Date information was recorded in an external archive

[44] MA_ExternalArchive_sID OPTIONAL,	-- Identifier of the external archive

[45] MA_ExternalArchiveSubmission OPTIONAL,	-- Date of submission of information in an external archive by a party

[46] MA_SpatialSourceType OPTIONAL,	-- The type of spatial source document

[47] MA_AdminSourceType OPTIONAL,	-- The type of administrative source document

[48] beginLifespanVersion,	-- Start time of a specific instance version

[49] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    -------------------------------------------------------------------------------

S121__AdministrativeSourceRecord

::= SEQUENCE {

SEQUENCE	{AdministrativeSourceRecordNamespace,	-- Unique Identifier for the Record instance

RecordNumbNumericID}

AdministrativeSourceTypeIdentifier,	-- Unique identifier of the type of record

AdministrativeSourceRecordContent,	-- Set of attributes pertaining to the specific record type

}

AdministrativeSourceNamespace 	::= Graphic String {”Source-”}

AdministrativeSourceTypeIdentifier 	::= Graphic String {”Administrative Source”}	-- Type of record taken from the “name” field for the type from the Feature Catalogue or described from the generic Location type in the Feature Catalogue..

AdministrativeSourceContent

::= SEQUENCE {

[1] MA_AdministrativeSourceType OPTIONAL,	-- The type of administrative source document

[2] SEQUENCE OF {MA_SourceORlinkageURL} OPTIONAL,	-- Location for online access

[3] SEQUENCE OF {MA_SourceORprotocol OPTIONAL,	-- Online Resource Protocol

[4] SEQUENCE OF {MA_SourceORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

[5] MA_SourceORname OPTIONAL,	-- Name of the resource

[6] MA_SourceORdescription OPTIONAL,	-- Description of what the resource is/does

[7] MA_SourceORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

[8] MA_RegistryNumber OPTIONAL,	-- Unique official identifier of the record in a registry

[9] MA_lifeSpanStamp OPTIONAL,	-- The moment that the event represented by the instance of LA_Source is further processed in the LA system

[10] MA_acceptance OPTIONAL,	-- The date of force of law of the source by an authority

[11] MA_maintype OPTIONAL,	-- The physical type of document

[12] MA_availabilityStatus OPTIONAL,	-- The availability status of document

[13] MA_submission OPTIONAL,	-- The date of submission of the source by a party

[14] MA_recordation OPTIONAL,	-- The date of registration (recordation) of the source by registering authority

[15] MA_RPindividualName OPTIONAL,	-- Name of the responsible individual

[16] MA_RPcontactORlinkageURL,	-- Responsible party location for online access

[17] MA_RPcontactORprotocol OPTIONAL,	-- Responsible party connection protocol to be used

[18] MA_RPcontactORapplicationProfile OPTIONAL,	-- Name of an application profile that can be used with the resource

[19] MA_RPcontactORname OPTIONAL,	-- Name of resource

[20] MA_RPcontactORdescription OPTIONAL,	-- Description of what the resource is/does

[21] MA_RPcontactORfunction OPTIONAL,	-- Function performed by the resource: download, information, offline access, order, search

[22] MA_RPorganizationName OPTIONAL,	-- Name of the responsible organization

[23] MA_RPpositionName OPTIONAL,	-- Position of the responsible person

[24] MA_RPcontactPhone OPTIONAL,	-- Telephone numbers at which the organisation or individual may be contacted

[25] MA_RProle OPTIONAL,	-- Function performed by the responsible party

[26] MA_RPcontactAddressCountry OPTIONAL,	-- Country of the physical address

[27] MA_RPcontactAddressDeliveryPoint OPTIONAL,	-- Address line for the physical address (Street name, box number, suite)

[28] MA_RPcontactAddressCity OPTIONAL,	-- City of the physical address

[29] MA_RPcontactElectronicMailAddress OPTIONAL,	-- Electronic Mail address

[30] MA_RPcontactAddressAdministrativeArea OPTIONAL,	-- State, province of the physical address

[31] MA_RPcontactAddressPostalCode OPTIONAL,	-- Postal Code for the physical address

[32] SEQUENCE OF{DQnameOfMeasure} OPTIONAL,	-- Data Quality name of measure

[33] DQmeasureIdentification OPTIONAL,	-- Data Quality measure identification

[34] DQmeasureDescription OPTIONAL,	-- Data Quality measure description

[35] DQevaluationMethodType OPTIONAL,	-- Data Quality evaluation method type

[36] DQevaluationMethodDescription OPTIONAL,	-- Data Quality evaluation method description

[37] DQevaluationProcedure OPTIONAL,	-- Data Quality evaluation method type

[38] SEQUENCE OF {DQdateTime} OPTIONAL,	-- Data Quality reference date

[39] SEQUENCE OF {DQresult},	-- Data Quality result

[40] MA_ExternalArchiveAcceptance OPTIONAL,	-- The date of force of law of the source by the authority

[41] MA_ExternalArchiveData OPTIONAL,	-- The content of the source

[42] MA_ExternalArchiveExtraction OPTIONAL,	-- Date of retrieval of information from an external archive

[43] MA_ExternalArchiveRecordation OPTIONAL,	-- Date information was recorded in an external archive

[44] MA_ExternalArchive_sID OPTIONAL,	-- Identifier of the external archive

[45] MA_ExternalArchiveSubmission OPTIONAL,	-- Date of submission of information in an external archive by a party

[46] MA_SpatialSourceType OPTIONAL,	-- The type of spatial source document

[47] MA_AdminSourceType OPTIONAL,	-- The type of administrative source document

[48] beginLifespanVersion,	-- Start time of a specific instance version

[49] endLifespanVersion OPTIONAL,	-- End time of a specific instance version

}

--    Attribute Types ---------------------------------------------------------------

-- Describes the type of the feature from a code list.

MA_Type	::= SEQUENCE {Graphic String {”Feature usage =”}

S121_FeatureType_Codelist_Characterstring

}

--    --------------------------------------------

-- A short textual identifier of the feature unit (e.g. a code)

MA_Label	::= SEQUENCE {Graphic String {”Feature code =”}

Graphic String

}

--    --------------------------------------------

-- describes the legal or administrative aspects of the feature object.

MA_Context	::= SEQUENCE {Graphic String {”Feature context =”}

Graphic String

}

--    --------------------------------------------

-- Describes the releasability of the feature.

MA_ Releasability

::= SEQUENCE {Graphic String {”Releasability =”}

Graphic String

}

--    --------------------------------------------

-- Begin time of a specific instance version

beginLifespanVersion

::= SEQUENCE {Graphic String {” Beginning of validity =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- End time of a specific instance version

endLifespanVersion

::= SEQUENCE {Graphic String {” End of validity =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- Describes the location of the feature as a textual statement.

MA_locationByText

::= SEQUENCE {Graphic String {” Location by text =”}

Graphic String

}

--    --------------------------------------------

-- Allows a CoordinateReferencingSystem (CRS) to optionally be specified.

MA_referenceSystem

::= SEQUENCE {Graphic String {” Coordinate Reference System =”}

RS_Identifier_StructuredCharacterString

}

--    --------------------------------------------

-- Maximum scale at which the data can be used.

scaleMaximum

::= SEQUENCE {Graphic String {” Maximum scale =”}

Graphic String

}

--    --------------------------------------------

-- Minimum scale at which the data can be used.

scaleMinimum

::= SEQUENCE {Graphic String {” Minimum scale =”}

Graphic String

}

--    --------------------------------------------

-- Computational origin of the element: defined, densified, computed or construction.

MA_orgtype	::= SEQUENCE {Graphic String {” Computational origin =”}

MLBOriginator_Type_Codelist_CharacterString

}

--    --------------------------------------------

-- The role of point in the structure of a straight line or curve.

MA_interpolationRole

::= SEQUENCE {Graphic String {” Interpolation role =”}

LA_InterpolationType_Codelist_CharacterString

}

--    --------------------------------------------

-- The type of point.

MA_pointType

::= SEQUENCE {Graphic String {” Type of point =”}

LA_PointType_Codelist_CharacterString

}

--    --------------------------------------------

-- Location of transformed point.

MA_TransformedLocation

::= SEQUENCE {Graphic String {” Adjusted location =”}

DirectPosition_FormattedCharacterString

}

--    --------------------------------------------

-- Type of computation used to define an arc (line) geodesic or loxodrome.

MA_arctyp	::= SEQUENCE {Graphic String {” Type of arc =”}

MLBArcType_Codelist_CharacterString

}

--    --------------------------------------------

-- Category of vertical domain of the object Any particular object may span more than one vertical domain.

MA_verdom	::= SEQUENCE {Graphic String {” Vertical domain =”}

Sequence Of {MLBJurDom_Type_Codelist_CharacterString}

}

--    --------------------------------------------

-- The area value.

MA_areaSize

::= SEQUENCE {Graphic String {” Area size =”}

Graphic String

}

--    --------------------------------------------

-- Area type: official area, non-official area, calculated area, surveyed area.

MA_areaType

::= SEQUENCE {Graphic String {” Area type =”}

LA_AreaType_Codelist_CharacterString

}

--    --------------------------------------------

-- The coordinates of a point inside the spatial unit.

MA_referencePoint

::= SEQUENCE {Graphic String	{” Reference point =”}

RS_Identifier_StructuredCharacterString

}

--    --------------------------------------------

-- Relationship to surface

MA_surfaceRelation

::= SEQUENCE {Graphic String {” Surface relationship =”}

LA_SurfaceRelationType_Codelist_CharacterString

}

--    --------------------------------------------

-- Unit of measure

MA_UnitOfMeasure

::= SEQUENCE {Graphic String {” Unit of measure =”}

Graphic String

}

--    --------------------------------------------

-- The volume of the space

MA_volumeSize

::= SEQUENCE {Graphic String {” Volume size =”}

Graphic String

}

--    --------------------------------------------

-- The volume type

MA_volumeType

::= SEQUENCE {Graphic String {” Volume Type =”}

LA_VolumeType_codelist_CharacterString

}

--    --------------------------------------------

-- GM_Point geometry as per IHO S100 V2 Part 7 Spatial Schema

MA_PointGeometry

::= SEQUENCE {Graphic String {” Position =”}

DirectPosition_FormattedCharacterString

}

--    --------------------------------------------

-- GM_OrientableCurve geometry as per IHO S100 V2 Part 7 Spatial Schema. Spatial primitives limited to GM_Curve, and GM_Curve (arcByCentrePoint and circleByCentrePoint) as per S-100.

MA_CurveGeometry

::= SEQUENCE {Graphic String {” Line =”}

PolyLineOrArc

}

--    --------------------------------------------

-- Start level for vertical extent

MA_VerticalPosition

::= SEQUENCE {Graphic String {” Vertical reference level =”}

DirectPosition_FormattedCharacterString

}

--    --------------------------------------------

-- Vertical extent of space

MA_VerticalExtent

::= SEQUENCE {Graphic String {” Vertical Extent =”}

Graphic String

}

--    --------------------------------------------

-- MA_BAname optionally establishes name for an instance of a BAUnit

MA_BAname	::= SEQUENCE {Graphic String {” Administrative unit type =”}

Graphic String

}

--    --------------------------------------------

-- The attribute MA_BAtype describes the use type of a BAUnit

MA_BAtype	::= SEQUENCE {Graphic String {” Administrative unit name =”}

S121_BAUnitType_Codelist_CharacterString

}

--    --------------------------------------------

-- The attribute MA_BAcontext allows the context for an instance of a BAUnit to be described

MA_BAcontext

::= SEQUENCE {Graphic String {” Administrative unit context =”}

Graphic String

}

--    --------------------------------------------

-- Description regarding the right, restriction or responsibility

MA_RRRdescription

::= SEQUENCE {Graphic String

{” Rights Restrictions Responsibility description =”}

Graphic String

}

--    --------------------------------------------

-- A share in an instance of a subclass of RRR

MA_RRRshare

::= SEQUENCE {Graphic String {” Share =”}

Fraction

}

--    --------------------------------------------

-- Boolean indicating whether the constraint is applicable

MA_RRRshareCheck

::= SEQUENCE {Graphic String {” Share constraint applicable =”}

Boolean

}

--    --------------------------------------------

-- Operational use of a right in time sharing

MA_RRRtimeSpec

::= SEQUENCE {Graphic String {” Share of a right in time =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- The type of the right

MA_RightType

::= SEQUENCE {Graphic String {” Right type =”}

S121_RightType_Codelist_CharacterString

}

--    --------------------------------------------

-- The type of the restriction

MA_RestrictionType

::= SEQUENCE {Graphic String {” Restriction type =”}

S121_RestrictionType_Codelist_CharacterString

}

--    --------------------------------------------

-- Boolean indicating whether a party is required for the registration of the restriction in the association to LA_Party

MA_partyRequired

::= SEQUENCE {Graphic String {” Party Required =”}

Boolean

}

--    --------------------------------------------

-- The type of the responsibility

MA_ResponsibilityType

::= SEQUENCE {Graphic String {” Responsibility type =”}

S121_ResponsibilityType_Codelist_CharacterString

}

--    --------------------------------------------

-- The identifier of the party in an external registration

extPID	::= SEQUENCE {Graphic String {” External party identifier =”}

Oid_FormattedCharacterString

}

--    --------------------------------------------

-- The name of the party

MA_partyName

::= SEQUENCE {Graphic String {” Party name =”}

Graphic String

}

--    --------------------------------------------

-- The role of the party

MA_ partyRole

::= SEQUENCE {Graphic String {” Party role =”}

Graphic String

}

--    --------------------------------------------

-- The type of the party

MA_partyType

::= SEQUENCE {Graphic String {” Party type =”}

S121_PartyType_Codelist_CharacterString

}

--    --------------------------------------------

-- The fraction of the whole for a party member

MA_partyShare

::= SEQUENCE {Graphic String {” Group Party Member Share =”}

Fraction

}

--    --------------------------------------------

-- The type of the group party

MA_groupPartyType

::= SEQUENCE {Graphic String {” Group party type =”}

S121_GroupPartyType_Codelist_CharacterString

}

--    --------------------------------------------

-- Document name - for example the document (legislation, treaty, title) that defines the object

MA_name	::= SEQUENCE {Graphic String {” Source name =”}

Graphic String

}

--    --------------------------------------------

-- Location for online access

MA_SourceORlinkageURL

::= SEQUENCE {Graphic String {” Source online locator =”}

Graphic String

}

--    --------------------------------------------

-- Online Resource Protocol

MA_SourceORprotocol

::= SEQUENCE {Graphic String {” Source online protocol =”}

Graphic String

}

--    --------------------------------------------

-- Name of an application profile that can be used with the resource

MA_SourceORapplicationProfile

::= SEQUENCE {Graphic String {” Source application profile =”}

Graphic String

}

--    --------------------------------------------

-- Name of the resource

MA_SourceORname

::= SEQUENCE {Graphic String {” Source name =”}

Graphic String

}

--    --------------------------------------------

-- Description of what the resource is/does

MA_SourceORdescription

::= SEQUENCE {Graphic String  {” Source description =”}

Graphic String

}

--    --------------------------------------------

-- Function performed by the resource: download, information, offline access, order, search

MA_SourceORfunction

::= SEQUENCE {Graphic String {” Source function =”}

CI_OnLineFunction_Codelist_Characterstring

}

--    --------------------------------------------

-- Unique official identifier of the record in a registry.

MA_RegistryNumber

::= SEQUENCE {Graphic String {” Source registry number =”}

Graphic String

}

--    --------------------------------------------

-- The moment that the event represented by the instance of LA_Source is further processed in the LA system

MA_lifeSpanStamp

::= SEQUENCE {Graphic String {” Source processed date =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- The date of force of law of the source by an authority

MA_acceptance

::= SEQUENCE {Graphic String  {” Source acceptance date =”}

Graphic String

}

--    --------------------------------------------

-- The physical type of document

MA_maintype

::= SEQUENCE {Graphic String  {” Source physical type =”}

CI_PresentationFromCode_Structured)CharacterString

}

--    --------------------------------------------

-- The availability status of document

MA_availabilityStatus

::= SEQUENCE {Graphic String {” Source availability status =”}

LA_AvailabilityStatusType_Codelist_CharacterString

}

--    --------------------------------------------

-- The date of submission of the source by a party

MA_submission

::= SEQUENCE {Graphic String  {” Source submission date =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- The date of registration (recordation) of the source by registering authority

MA_recordation

::= SEQUENCE {Graphic String {” Source date of recordation =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- Name of the responsible individual

MA_RPindividualName

::= SEQUENCE {Graphic String {” Responsible party name =”}

Graphic String

}

--    --------------------------------------------

-- Responsible party location for online access

MA_RPcontactORlinkageURL

::= SEQUENCE {Graphic String {” Responsible party online contact =”}

Graphic String

}

--    --------------------------------------------

-- Responsible party connection protocol to be used

MA_RPcontactORprotocol

::= SEQUENCE {Graphic String {” Responsible party online protocol =”}

Graphic String

}

--    --------------------------------------------

-- Name of an application profile that can be used with the resource

MA_RPcontactORapplicationProfile

::= SEQUENCE {Graphic String {” Responsible party application profile =”}

CI_OnLineFunction_Codelist_Characterstring

}

--    --------------------------------------------

-- Name of resource

MA_RPcontactORname

::= SEQUENCE {Graphic String {” Responsible party resource description =”}

Graphic String

}

--    --------------------------------------------

-- Description of what the resource is/does

MA_RPcontactORdescription

::= SEQUENCE {Graphic String {” Responsible party resource name =”}

Graphic String

}

--    --------------------------------------------

-- Function performed by the resource: download, information, offline access, order, search

MA_RPcontactORfunction

::= SEQUENCE {Graphic String {” Responsible party resource function =”}

Graphic String

}

--    --------------------------------------------

-- Name of the responsible organization

MA_RPorganizationName

::= SEQUENCE {Graphic String {” Responsible party organization name =”}

Graphic String

}

--    --------------------------------------------

-- Position of the responsible person

MA_RPpositionName

::= SEQUENCE {Graphic String

{” Responsible party position =”}

Graphic String

}

--    --------------------------------------------

-- Telephone numbers at which the organisation or individual may be contacted

MA_RPcontactPhone

::= SEQUENCE {Graphic String {” Responsible party contact phone =”}

Graphic String

}

--    --------------------------------------------

-- Function performed by the responsible party

MA_RProle	::= SEQUENCE {Graphic String {” Responsible party role =”}

CI_RoleCode_Codelist_CharacterString

}

--    --------------------------------------------

-- Country of the physical address

MA_RPcontactAddressCountry

::= SEQUENCE {Graphic String {” Responsible party address country =”}

Graphic String

}

--    --------------------------------------------

-- Address line for the physical address (Street name, box number, suite)

MA_RPcontactAddressDeliveryPoint

::= SEQUENCE {Graphic String {” Responsible party delivery point =”}

Graphic String

}

--    --------------------------------------------

-- City of the physical address

MA_RPcontactAddressCity

::= SEQUENCE {Graphic String {” Responsible party city =”}

Graphic String

}

--    --------------------------------------------

-- Electronic Mail address

MA_RPcontactElectronicMailAddress

::= SEQUENCE {Graphic String {” Responsible party electronic mail address =”}

Graphic String

}

--    --------------------------------------------

-- State, province of the physical address

MA_RPcontactAddressAdministrativeArea

::= SEQUENCE {Graphic String {” Responsible party state/province =”}

Graphic String

}

--    --------------------------------------------

-- Postal Code for the physical address

MA_RPcontactAddressPostalCode

::= SEQUENCE {Graphic String {” Responsible party postal code =”}

Graphic String

}

--    --------------------------------------------

-- Data Quality name of measure

DQnameOfMeasure

::= SEQUENCE {Graphic String {” Data Quality name of measure =”}

Graphic String

}

--    --------------------------------------------

-- Data Quality measure identification

DQmeasureIdentification

::= SEQUENCE {Graphic String {” Data Quality measure identification =”}

Graphic String

}

--    --------------------------------------------

-- Data Quality measure description

DQmeasureDescription

::= SEQUENCE {Graphic String {” Data Quality measure description =”}

Graphic String

}

--    --------------------------------------------

-- Data Quality evaluation method type

DQevaluationMethodType

::= SEQUENCE {Graphic String {” Data Quality evaluation method type =”}

DQ_EvaluationTypeCode_Codelist_CharacterString

}

--    --------------------------------------------

-- Data Quality evaluation method description

DQevaluationMethodDescription

::= SEQUENCE {Graphic String

{” Data Quality evaluation method description =”}

Graphic String

}

--    --------------------------------------------

-- Data Quality evaluation method type

DQevaluationProcedure

::= SEQUENCE {Graphic String {” Data Quality evaluation method type =”}

Graphic String

}

--    --------------------------------------------

-- Data Quality reference date

DQdateTime	::= SEQUENCE {Graphic String {” Data Quality reference date =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- Data Quality result

DQresult	::= SEQUENCE {Graphic String {” Data Quality result =”}

Graphic String

}

--    --------------------------------------------

-- The date of force of law of the source by the authority

MA_ExternalArchiveAcceptance

::= SEQUENCE {Graphic String {” External Archive date of acceptance =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- The content of the source

MA_ExternalArchiveData

::= SEQUENCE {Graphic String {” External Archive information =”}

Graphic String

}

--    --------------------------------------------

-- Date of retrieval of information from an external archive

MA_ExternalArchiveExtraction

::= SEQUENCE {Graphic String {” External Archive date of retrieval =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- Date information information was recorded in an external archive

MA_ExternalArchiveRecordation

::= SEQUENCE {Graphic String {” External Archive date of recordation =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- Identifier of the external archive

MA_ExternalArchive_sID

::= SEQUENCE {Graphic String {” External Archive Identifier =”}

Oid_FormattedCharacterString

}

--    --------------------------------------------

-- Date of submission of information in an external archive by a party

MA_ExternalArchiveSubmission

::= SEQUENCE {Graphic String {” External Archive date of submission =”}

DateTime_StructuredCharacterString

}

--    --------------------------------------------

-- The type of spatial source document

MA_SpatialSourceType

::= SEQUENCE {Graphic String {” Spatial source type =”}

Graphic String

}

--    --------------------------------------------

-- The type of administrative source document

MA_AdminSourceType

::= SEQUENCE {Graphic String {” Administrative source type =”}

S121_AdministrativeSourceType_Codelist_CharacterString

}

--   Code Lists ----------------------------------------------------------------

S121_FeatureType_Codelist_Characterstring

::= Choice {	[1]	Graphic String {”MLB”}

[2]	Graphic String {”A76”}

[3]	Graphic String {”otherFeatureType”}

}

--   --------------------------------------------

MLBOriginator_Type_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”defined”}

[2]	Graphic String {”densified”}

[3]	Graphic String {”computed”}

[4]	Graphic String {”construction”}

}

--   ---------------------------------------

LA_InterpolationType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”midArc”}

[2]	Graphic String {”start”}

[3]	Graphic String {”end”}

[4]	Graphic String {”mid”}

[5]	Graphic String {”isolated”}

}

--   ---------------------------------------

LA_PointType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”source”}

[2]	Graphic String {”control”}

[3]	Graphic String {”noSource”}

}

--   ---------------------------------------

MLBArcType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”geodesic”}

[2]	Graphic String {”loxodrome”}

}

--   ---------------------------------------

MLBJurDom_Type_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”airspace”}

[2]	Graphic String {”landSurface”}

[3]	Graphic String {”waterSurface”}

[4]	Graphic String {”waterColumn”}

[5]	Graphic String {”seabedSurface”}

[6]	Graphic String {”subsoil”}

}

--   ---------------------------------------

LA_SurfaceRelationType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”mid”}

[2]	Graphic String {”below”}

[3]	Graphic String {”above”}

[4]	Graphic String {”onSurface”}

}

--   ---------------------------------------

LA_VolumeType_codelist_CharacterString

::= Choice {	[1]	Graphic String {”officialVolume”}

[2]	Graphic String {”nonOfficialVolume”}

[3]	Graphic String {”calculatedVolume”}

[4]	Graphic String {”surveyedVolume”}

}

--   ---------------------------------------

S121_BAUnitType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”SovereigntyUnit”}

[2]	Graphic String {”SovereignRightsUnit”}

[3]	Graphic String {”JointDevelopmentArea”}

[4]	Graphic String {”OtherRegulatoryUnit”}

}

--   ---------------------------------------

S121_RightType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”sovereignty”}

[2]	Graphic String {”sovereignRight”}

[3]	Graphic String {”accessRight”}

[4]	Graphic String {”harvestRight”}

[5]	Graphic String {”easementRight”}

}

--   ---------------------------------------

S121_RestrictionType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”timeBasedRestriction”}

[2]	Graphic String {”passageRestriction”}

[3]	Graphic String {”accessRestriction”}

[4]	Graphic String {”useRestriction”}

[5]	Graphic String {”jurisdictionRestriction”}

[6]	Graphic String {”resourceRestriction”}

}

--   ---------------------------------------

S121_ResponsibilityType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”maintenanceResponsibility”}

}

--   ---------------------------------------

S121_PartyType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”naturalPerson”}

[2]	Graphic String {”nonNaturalPerson”}

[3]	Graphic String {”accessRestriction”}

[4]	Graphic String {”state-country”}

[5]	Graphic String {”province-state”}

[6]	Graphic String {”internationalOrganization”}

}

--   ---------------------------------------

S121_GroupPartyType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”agreement”}

[2]	Graphic String {”association”}

}

--   ---------------------------------------

CI_OnLineFunction_Codelist_Characterstring

::= Choice {	[1]	Graphic String {”download”}

[2]	Graphic String {”information”}

[3]	Graphic String {”offlineAccess”}

[4]	Graphic String {”order”}

[5]	Graphic String {”search”}

}

--   ---------------------------------------

LA_AvailabilityStatusType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”archiveConverted”}

[2]	Graphic String {”archiveDestroyed”}

[3]	Graphic String {”archiveIncomplete”}

[4]	Graphic String {”archiveUnknown”}

[5]	Graphic String {”docAvailable”}

}

--   ---------------------------------------

CI_RoleCode_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”resourceProvider”}

[2]	Graphic String {”custodian”}

[3]	Graphic String {”owner”}

[4]	Graphic String {”user”}

[5]	Graphic String {”distributor”}

[6]	Graphic String {”originator”}

[7]	Graphic String {”pointOfContact”}

[8]	Graphic String {”principleInvestigator”}

[9]	Graphic String {”processor”}

[10]	Graphic String {”publisher”}

[11]	Graphic String {”author”}

}

--   ---------------------------------------

DQ_EvaluationTypeCode_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”directInternal”}

[2]	Graphic String {”directExternal”}

[3]	Graphic String {”indirect”}

}

--   ---------------------------------------

S121_AdministrativeSourceType_Codelist_CharacterString

::= Choice {	[1]	Graphic String {”internationalAgreement”}

[2]	Graphic String {”domesticLegislativeInstrument”}

[3]	Graphic String {”domesticAgreement”}

}

--   ---------------------------------------

CI_PresentationFormCode_Structured_CharacterString

::= Choice {	[1]	Graphic String {”documentDigital”}

[2]	Graphic String {”documentHardcopy”}

[3]	Graphic String {”inageDigital”}

[4]	Graphic String {”imageHardcopy”}

[5]	Graphic String {”mapDigital”}

[6]	Graphic String {”mapHardcopy”}

[7]	Graphic String {”modelDigital”}

[8]	Graphic String {”modelHardcopy”}

[9]	Graphic String {”profileDigital”}

[10]	Graphic String {”profileHardcopy”}

[11]	Graphic String {”tableDigital”}

[12]	Graphic String {”tableHardcopy”}

[13]	Graphic String {”videoDigital”}

[14]	Graphic String {”videoHardcopy”}

}

--   Structured or Formatted Character Strings ------------------------------------------------

Oid_FormattedCharacterString

::= SEQUENCE {	GraphicString	-- namespace

GraphicString  {” - ”}	-- dashDelimiter

GraphicString	-- localId

}

--   ---------------------------------------

DirectPosition_FormattedCharacterString

::= SEQUENCE {	GraphicString 	-- Lat_position

GraphicString {” / ”}	-- SlashDelimiter

GraphicString	-- Long_position

}

--   ---------------------------------------

PolyLineOrArc

::= Choice {	[1]	S121_PolyLine

[2]	S121_ArcByCentrePoint

[3]	S121_CircleByCentrePoint

}

S121_PolyLine

::= SEQUENCE {Vertex_StructuredCharacterString

}

S121_ArcByCentrePoint     ::= ArcByCentrePointStructuredCharacterString

S121_CircleByCentrePoint  ::= CircleByCentrePointStructuredCharacterString

ArcByCentrePointStructuredCharacterString

::= SEQUENCE {	GraphicString {” ArcStart_Lat/Long ”}	-- StartIndicator Delimiter

GraphicString	-- Start_Lat_Position

GraphicString {” / ”}	-- SlashDelimiter

GraphicString	-- Start_Long_Position

GraphicString {” ArcEnd_Lat/Long ”}	-- EndIndicator Delimiter

GraphicString	-- End_Lat_Position

GraphicString {” / ”}	-- SlashDelimiter

GraphicString	-- End_Long_Position

GraphicString {” ArcCentre_Lat/Long ”}	-- CentreIndicator Delimiter

GraphicString	-- Centre_Lat_Position

GraphicString {” / ”}	-- SlashDelimiter

GraphicString	-- Centre_Long_Position

}

CircleByCentrePointStructuredCharacterString

::= SEQUENCE {	GraphicString {” CircelCentre_Lat/Long ”}	-- CentreIndicator Delimiter

GraphicString	-- Centre_Lat_Position

GraphicString {” / ”}	-- SlashDelimiter

GraphicString	-- Centre_Long_Position

GraphicString {” CircelPoint_Lat/Long ”}	-- CircelPointIndicator Delimiter

GraphicString	-- CircelPoint_Lat_Position

GraphicString {” / ”}	-- SlashDelimiter

GraphicString	-- CircelPoint_Long_Position

}

--   ---------------------------------------

-- Structured character string representing date and time.

-- The format is    YYYY:MM:DD:hh:mm:ss:timeZone

-- The sub fields are separated by the delimiter  :

--    YYYY is the year    e.g.  2017

--    MM is the month  e.g. 05   (May)

--    DD is the day  e.g. 17

--    hh is the hour  e.g. 13   (in 24 hour clock)

--    mm is the minutes e.g.  15

--    ss is the seconds  e.g.   00

--    timeZone is a shift from Greenwich Mean Time in hours

--    e.g. -5 for Eastern North America (EST).

--    The plus sign + is optional for east of Greenwich.

--

-- Therefore a date/time would be   2017:05:17:13:15:00:-5

-- Any unknown fields may be represented by consecutive delimiters e.g.   ::

DateTime_StructuredCharacterString

::= SEQUENCE {	GraphicString OPTIONAL	-- centuaryAndYear

GraphicString OPTIONAL {”:”}	-- ColonDelimiter

GraphicString OPTIONAL	-- month

GraphicString OPTIONAL {”:”}	-- ColonDelimiter

GraphicString OPTIONAL	-- day

GraphicString OPTIONAL {”:”}	-- ColonDelimiter

GraphicString OPTIONAL	-- hour

GraphicString OPTIONAL {”:”}	-- ColonDelimiter

GraphicString OPTIONAL	-- minute

GraphicString OPTIONAL {”:”}	-- ColonDelimiter

GraphicString OPTIONAL	-- second

GraphicString OPTIONAL {”:”}	-- ColonDelimiter

GraphicString OPTIONAL	-- timeZone

}

--   ---------------------------------------

RS_Identifier_StructuredCharacterString

::= SEQUENCE {	GraphicString	-- codeSpace

GraphicString	-- version

}

--   ---------------------------------------

-- In an explicit text record oriented implementation a Distance data type will be represented

-- as the text string  representing a distance as a number in the Units of Measure defined in

-- the MA_UnitOfMeasure attribute (or in metadata).

Distance	::= GraphicString

--   ---------------------------------------

-- In an explicit text record oriented implementation a Fraction data type will be represented

-- as the text string "0.xxxx..." where xxx... is the mantissa (fractional part of a number).

Fraction	::= GraphicString

--   ---------------------------------------

-- In an explicit text record oriented implementation a Boolean data type will be represented

-- as the text Yes or No.

Boolean	::= GraphicString

--   ---------------------------------------

ISO 19136 Geographic information -- Geography Markup Language (GML) and OGC OpenGIS Geography Markup Language (GML) Encoding Standard version 3.2.1 OGC # 07-036

Extensible Markup Language (XML), World Wide Web Consortium W3C < https://www.w3.org/TR/REC-xml/ >

ISO 19128 Geographic information – Web map service interface (WMS) and OGC OpenGIS Web Map Service (WMS) Implementation Specification OGC # 06-042

ISO 19142 Geographic information -- Geography feature service (WFSL) and OGC OpenGIS Web Feature Service 2.0 Interface Standard OGC # 09-025r2

OGC OpenGIS OGC® Catalogue Services 3.0 OGC # 12-168r6, 12-176r7, 07-006r1

Keyhole Markup Language (KML) OGC # 12-007r2

ISO/IEC 8824-1:2015 Information technology -- Abstract Syntax Notation One (ASN.1)

IA5 is the International Alphabet number 5 standardized as ISO standard 646 International Reference Version and as a UN International Telecommunications Union recommendation T.50. It is equivalent to the US American Standard Code for Information Interchange (ASCII) except for the currency symbol $ which may be substituted in some nations for a different currency symbol. IA5 is also the base of the UTF-8 character set used widely on the internet and the base page of Unicode (ISO 10646) multilingual character set.

https://en.wikipedia.org/wiki/Hex_dump

The TSV structure is not fully standardized. To comply with Microsoft’s rules the LF character may be used within a character string and this block of text will be read into one cell in the Excel spreadsheet program. However Unix and Apple do not use LF in the same way. Nevertheless the S-121 Explicit Text Format is unambiguous and can always be parsed or printed with the same results. Best practice is to follow the Microsoft rule and use LF when necessary to separate “paragraphs” within a text string in an attribute value.

This is the same mechanism used in XML, but the selection of delimiters is different.

The ISO standard character code includes a set of unprintable delimiter's, and these have been used in a number of record oriented data formats. These unprintable "invisible" characters cannot be used here because a human reader cannot see them and would perceive that something was going on "behind in the background". Everything must be explicit in a legal environment. TAB has a visual effect and therefore can be seen.

This issue is common in many commercial software packages where it is difficult to include more than one line of text in, for example, a cell in a spread sheet.

The Metadata entry “Metadata - S121 identifier” is an S-100 required metadata field. The contents are “S-121”. The expression of this field in a dataset is optional and as such would likely never be used.