- Test Dataset: AA400017

S‐58 Recommended ENC Validation Checks covered in this section:

| S‐58 Check | Description |
| --- | --- |
| 99 | For SG3D that contains > 8250 3D coordinates |
| 100 | For SG2D that contains > 12375 2D coordinates |
| 101 | For FSPT that references > 12375 VRID records |

Secondary Errors

Critical – 505, 1726

Error – 81, 549

Warnings – 19, 515, 571

| Dataset Name | AA400017 |  |  | S‐58 Check |  | 99 |  |  | Type | C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-58 Description | For SG3D that contains > 8250 3D coordinates |  |  |  |  |  |  |  |  |  |
| Message | Sounding bundle contains more than 8250 individual depths |  |  |  |  |  |  |  |  |  |
| Solution | Split sounding references SG3D. |  |  |  |  | Conformity |  | Part 3 (2.7). |  |  |
| Test Case No. 1 | SOUNDG feature referencing a SG3D record with 8281 3D coordinates. |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |
| 32°28'S 61°18'E |  | SOUNDG | QUASOU = 1; TECSOU = 14; SCAMIN = 59999 |  | FE-06 |  | AA 0020018805 00002 |  |  | VI-01 |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 99: 1 errors “Sounding bundle contains more than 8250 individual depths” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None |  |  |  |  |  |  |  |  |  |
| Dataset Name | AA400017 |  |  | S‐58 Check |  | 100 |  |  | Type | C |
| S-58 Description | For SG2D that contains > 12375 2D coordinates. |  |  |  |  |  |  |  |  |  |
| Message | Edge contains more than 12375 coordinates. |  |  |  |  |  |  |  |  |  |
| Solution | Split or optimise geometry. |  |  |  |  | Conformity |  | Part 3 (2.7) . |  |  |
| Test Case No. 1 | ROADWY references a SG2D edge with 12385 coordinates. |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |
| 32°28'19"S 61°22'11"E |  | ROADWY(L) | CATROD = 2; STATUS = 1; NATCON = 4; |  | FE-03 |  | AA 0020018802 00002 |  |  | VE-12383 |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 100: An error “Edge contains more than 12375 coordinates” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None. |  |  |  |  |  |  |  |  |  |
| Dataset Name | AA400017 |  |  | S‐58 Check |  | 101 |  |  | Type | C |
| S-58 Description | For FSPT that references > 12375 VRID records. |  |  |  |  |  |  |  |  |  |
| Message | FSPT record contains references to more than 12375 spatial records. |  |  |  |  |  |  |  |  |  |
| Solution | Optimise geometry, merging referenced edges where possible . |  |  |  |  | Conformity |  | Part 3 (2.7) |  |  |
| Test Case No. 1 | COALNE (L) feature references 12380 edges. |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |
| 32°27'30"S 61°18'22"E |  | COALNE (L) | CATCOA=5 |  | FE-05 |  | AA 0020018804 00002 |  |  | VE-01 to VE-12380; |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101 : error “FSPT record contains references to more than 12375 spatial records” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None. |  |  |  |  |  |  |  |  |  |
| Test Case No. 2 | LNDARE (A) feature references 12381 edges.. |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |
| 32°26'S 61°18'E |  | LNDARE (A) |  |  | FE-02 |  | AA 0020018804 00002 |  |  | VE-01 to VE-12381; |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101 : error “FSPT record contains references to more than 12375 spatial records” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None. |  |  |  |  |  |  |  |  |  |
| Test Case No. 3 | DEPARE (A) feature references 12381 edges.. |  |  |  |  |  |  |  |  |  |
| Location |  | Feature | Attributes |  | FRID |  | FOID |  |  | VRID |
| 32°28'S 61°18'E |  | DEPARE (A) | DRVAL1 = 0; DRVAL2 = 10; QUASOU = 1; |  | FE-04 |  | AA 0020018803 00002 |  |  | VE-01 to VE-12380; VE-12382; |
| Screen Capture |  |  |  |  |  |  |  |  |  |  |
| Expected Test Results | 101 : error “FSPT record contains references to more than 12375 spatial records” must be triggered. |  |  |  |  |  |  |  |  |  |
| Secondary Critical Errors | None. |  |  |  |  |  |  |  |  |  |